CSA-WFS-RD-0005

Canadian Space Agency

WildFireSat

General Design and Interface Requirements (GDIR) Initial Release January 15, 2024

This Page Intentionally Left Blank

Preface

This document must be approved by the undersigned.

| Prepared by: |  |  |
| --- | --- | --- |
|  | Geneviève Houde WildFireSat Systems Engineer Sun-Earth System Sciences, Space Utilization Directorate, CSA | Date |
| Prepared by: | WildFireSat Team |  |
| Approved by: |  |  |
|  | Isabelle Tremblay Executive Director, Space Infrastructure Space Utilization Directorate, CSA | Date |

Revision History

| Rev. | Description | Initials | Date |
| --- | --- | --- | --- |
| Draft 1 | Initial Release Released by the approval of Draft 1 | WFS Team | February 7, 2023 |
| Draft 2 | Added the following requirements, which were originally in older version of the Product Assurance Requirements document: WFS-GDIR-0955 WFS-GDIR-1215 | WFS Team | May 18, 2023 |
| Draft 3 | Modified requirements: WFS-GDIR-0110 WFS-GDIR-0310 WFS-GDIR-0320 WFS-GDIR-0340 WFS-GDIR-0360 WFS-GDIR-0370 WFS-GDIR-0640 Added a note to requirements: WFS-GDIR-0140 WFS-GDIR-0350 Deleted requirement: WFS-GDIR-1780 | WFS Team | October 18, 2023 |
| Draft 4 | Added a note in section 1.1 about priority of PAR vs GDIR Added a note addressed to WFS bidders in section 3 Moved requirements from EDTS to GDIR: WFS-GDIR-1201 WFS-GDIR-0181 Deleted requirements: WFS-GDIR-0040 WFS-GDIR-0260 WFS-GDIR-1500 Modified requirements: WFS-GDIR-0060 WFS-GDIR-0120 WFS-GDIR-0240 WFS-GDIR-0290 WFS-GDIR-0320 WFS-GDIR-1090 WFS-GDIR-1215 Added requirements: WFS-GDIR-0181 WFS-GDIR-1111 | WFS Team | October 31, 2023 |
| IR | Initial Release | WFS Team | January 15, 2024 |

Table of Contents

1.	Introduction	1

1.1	Purpose of This Document	1

1.2	Document Language Conventions	1

1.3	Formatting Conventions	2

1.4	Document Overview	2

2.	Applicable and Reference Documents	3

2.1	Applicable Documents	3

2.2	Reference Documents	4

3.	Space Segment General Design and Interface Requirements	5

3.1	Reference Frames	5

3.2	Margins	5

3.3	Mechanical Design and Interfaces	6

3.3.1	Equipment Mounting Provisions	6

3.3.2	Connector Requirements	9

3.3.3	Equipment Surfaces	11

3.3.4	Electrical Bonding	11

3.3.5	Contamination and Control Requirements	14

3.3.6	Alignment	14

3.3.7	Structural Design	15

3.3.8	Mechanisms	18

3.3.9	Material Properties and Allowables	23

3.3.10	Venting and Depressurization	24

3.4	Thermal Design and Interfaces	24

3.4.1	General	24

3.4.2	Thermal Hardware	24

3.4.3	Temperature Control	24

3.4.4	Thermal Interface	25

3.5	Electrical Design and Interfaces	27

3.5.1	Test Stimuli and Test Points Interfaces	27

3.5.2	Power Stability	29

3.5.3	Switch OFF Characteristics	29

3.5.4	Switch ON Characteristics	30

3.5.5	Operational Transients	30

3.5.6	Protection	30

3.5.7	Voltage Tolerance	32

3.5.8	Power	33

3.5.9	Part Stress and Derating	35

3.6	TT&C related requirements	35

3.6.1	General Requirements	35

3.7	Software Design and Interfaces	38

3.8	Wiring Harness Design and Interfaces	40

3.9	Launch Vehicle Interface	42

3.10	Electrostatic Discharge Control	44

3.11	Intentional Transmitters and Receivers	45

3.12	Multipaction	46

3.13	EMC/EMI	47

4.	Acronyms, Abbreviations and Definitions	48

4.1	Acronyms and Abbreviation	48

4.2	Definitions	51

Appendices	53

List of Figures

FIGURE	PAGE

Figure 3-1 – Temperature Margins (Reference AD-05)	27

List of Tables

TABLE	PAGE

Table 3-1 – Maximum Bonding Impedances	12

Table 3-2 – Bond – Connections of Multi-Layer Insulation	13

Table 3-3 – Design Safety Factors – General	15

Table 3-4 – Design Safety Factors – Sandwich Panels	16

Table 3-5 – Design Safety Factors – Composites	16

Table 3-6 – Design Effects Requiring Additional Design Factors	17

# Introduction

## Purpose of This Document

This document defines the General Design and Interface Requirements (GDIR) applicable to the WildFireSat mission.

The content of this document is based on well established and recognized space domain best practices and are highly influenced by NASA and European Cooperation for Space Standardization (ECSS) standards as well as CSA lessons learned from previous missions.

AD-01 has priority over this document in case of conflicts.

## Document Language Conventions

The following words, as used in this document, have specific meaning as indicated below:

| shall | indicates a mandatory requirement; |
| --- | --- |
| should | indicates a preferred but not mandatory alternative; |
| will | indicates a statement of intention or fact; |
| may | indicates an option. |

Notes are for information to help clarify the requirements.

## Formatting Conventions

The identifier for each WFS GDIR requirements is of the structure:

[WFS-GDIR-{ID}]

Where:

{ID} is a sequential 4-digit number (with leading zeros as required) corresponding to the requirement for the given category.

Each requirement in this document consists of the following components:

A unique identifier as described above.

A descriptive title, which is informative and not part of the formal requirement statement.

The formal requirement statement.

Rationale: provide information on the rationale for the requirement.

- In some cases, notes are added before or after a requirement or goal.  The notes provide background information to aid in the understanding of the requirement or goal.  Notes are italized.

## Document Overview

This Document is structured as follows:

Section 1 provides the purpose and structure of the document.

Section 2 lists the applicable and reference documentation.

Section 3 defines the GDIR requirements

Section 4 Acronyms, Abbreviations and Definitions

Appendix A describes the margin philosophy

# Applicable and Reference Documents

## Applicable Documents

The following documents of the exact issue date and revision level shown are applicable and form an integral part of this document to the extent specified herein.

In the event that the Contractor has recognized a conflict between requirements and/or conditions stated in the applicable documents, such conflict must be identified to the CSA for clarification.

| AD | Document Number | Revision | Title |
| --- | --- | --- | --- |
|  | CSA-SMA-RD-0008 | A | Generic Product Assurance Requirements (PAR) – Class B |
|  | Deleted |  |  |
|  | Deleted |  |  |
|  | Deleted |  |  |
|  | ARC-STD-8070.1 | Dec 8, 2018 | NASA Ames Technical Standard Space Flight System Design and Environmental Tests |
|  |  | 2020 | ITU Radio Regulations |
|  | Deleted |  |  |
|  | Deleted |  |  |
|  | ECSS-E-HB-32-23A | 16 April 2010 | Threaded fasteners handbook |
|  | ECSS-E-HB-32-22A | 20 March 2011 | Insert design handbook |
|  | ECSS-E-ST-33-01C | Rev. 2 1 March 2019 | Mechanisms |
|  | MIL-G-45204 | C | Military Specification: Gold Plating, Electrodeposited |
|  | Deleted |  |  |
|  | Deleted |  |  |
|  | ECSS-E-ST-32-02C | Rev. 1 15 Nov 2008 | Structural design and verification of pressurized hardware |
|  | JSC-CR-06-070 |  | Space Vehicle RF Environments |
|  | ANSI/AIAA S-080A | 2018 | Space Systems – Metallic Pressure Vessels, Pressurized Structures and Pressure Components |
|  | ANSI/AIAA S-081B | 2018 | Space Systems – Composite Overwrapped Pressure Vessels |
|  | CSA-WFS-RD-0006 | IR | WFS Environmental Design and Test Specification (EDTS) |
|  | ECSS-E-ST-20-07C | Rev.2 (January 2022) | Electromagnetic compatibility |

## Reference Documents

The following documents provide additional information or guidelines that either may clarify the contents or are pertinent to the history of this document.

| RD | Document Number | Revision | Title |
| --- | --- | --- | --- |
|  | ECSS-E-ST-20-01C | 15 June 2020 | ECSS Space Engineering Multipaction Design and Test |
|  | Deleted |  |  |
|  | ECSS-E-ST-20-06C | Rev. 1 15 May 2019 | Spacecraft Charging |
|  | NASA-STD-4003 | Rev. A | ELECTRICAL BONDING FOR NASA LAUNCH VEHICLES, SPACECRAFT, PAYLOADS, AND FLIGHT EQUIPMENT |
|  | NASA-HDBK-4002 | A | Mitigating In-Space Charging Effects-A Guideline |
|  | CSA-WFS-RD-0004 | IR | WFS System Requirements Document |

# Space Segment General Design and Interface Requirements

This section defines the general requirements for the design and the interfaces of the space segment.

NOTE TO WFS BIDDERS: When redlining the document as part of your proposal, ensure that you are comfortable meeting all the requirements in your redline version, and that they make sense with your concept, including your spacecraft size, and level of heritage. The redline version will be assessed against the residual risk to the mission, taking into account your concept and heritage.

## Reference Frames

- Reference Frames Definitions

The required reference frames (as a minimum one for spacecraft flight, one for spacecraft mechanical and one for the payload) for the design and AIT, launch and operations shall be defined, including, for each of them, a standard identifier, the location of the origin and the orientation of the main axes.

Rationale:	Various data are relative to reference frames and must be properly and consistently defined for ease and reliability of interpretation.

- Reference Frames Rotation Matrices

The rotation matrices linking each of the reference frames shall be defined.

Rationale:	Rotation matrices allow translation of one reference frame to another.

## Margins

- System Engineering Margins

The minimum system engineering margins for the WildFireSat flight segment development shall be as in Appendix A.

Rationale:	To ensure robustness and reliability of the design.

## Mechanical Design and Interfaces

### Equipment Mounting Provisions

- Unit MGSE Interface Points

Any flight equipment with a mass greater than 20 kg shall be provided with adequate GSE mechanical interface points for ground hanlding, lifting and integration into the spacecraft.

Rationale:	To support safe AI&T activities.

- Unit Mounting Bolts and Washers

The equipment shall be attached using flat washers and bolts calculated in accordance with ECSS-E-HB-32-23A (AD-09).

Rationale:	To standardize the analysis approach for threaded fastener joints.

- Deleted

- Equipment Access- Goal

Access for inspection, maintenance, removal and replacement of units inside the spacecraft should not be impeded by other spacecraft components (e.g. thermal hardware, harness…).

Rationale:	To minimize turnaround time in case of anomaly at spacecraft level.

- Equipment Access After Spacecraft Testing

Any access anticipated to be needed after spacecraft testing and prior to launch (e.g. umbilical and temporary connection protective covers) shall be possible without requiring dismantling the spacecraft, a unit, a panel or any piece of equipment.

Rationale:	To prevent invalidating vibration or the critical environment testing results and minimize turnaround time at the launch site.

- Balance Mass Access

Balance mass mounting areas shall be readily accessible until Flight Readiness Review (FRR).

Rationale:	To minimize turnaround time at the launch site.

- Minimum Attachment Hole Spacing

The minimum allowed distance between attachment holes for equipment on the structure shall be:

- 15 mm for mounting on solid machined structures; and

51 mm for mounting on honeycomb panels.

unless smaller distances are determined based on calculations using ECSS-E-HB-32-22A (AD-10).

Rationale:	Based on common design practices and to avoid proximity factor effects.

- Unpainted Spot Face Area for Washers

The units shall provide an unpainted spot face area under the washer for each mounting bolt.

Rationale:	To enable metal-to-metal contact.

- Mounting Interface Flatness and Roughness

The plane of the attachment feet, and any other interface surface nominally lying in the plane of those feet, for each equipment, shall be flat to within:

| - Mounting surface roughness | £ 3 µm |
| --- | --- |
| Local mounting surface flatness | £ 0.1 mm / 100 mm |
| Overall mounting surface flatness | £ 0.2 mm |
| Overall mounting surface flatness for equipment requiring alignment | £ 50 µm |

Rationale:	Based on common design practices.

- Number of Mounting Feet

The number of mounting feet (fixation points) per panel-mounted equipment shall be selected to ensure that at each mounting bolt, the assembly does not generate insert loads greater than the loads allowable when subjected to the qualification level quasi-static loads specified for the equipment.

Note: Verification of inserts loads allowable can be demonstrated by test or calculations based on ECSS-E-HB-32-22A(AD-10).

Rationale:	To ensure that the insert load remains inferior to the allowable under worst case test conditions.

- Locking Devices

Bolts and nuts shall be fastened by  at least one locking mechanism, such as strapping compound, locking inserts, etc on final assembly. Where there is only a mechanical locking device used, torque stripe is required (if staking is not used) on final assembly.

- Rationale:	Post vibration there is no way to verify if hardware pre-tension torque was exceeded.

- Locking Washers

Locking washers shall not be used on the spacecraft.

Rationale:	Method is proven to be unreliable and better methods can be easily implemented.

- Single Point Failure Fasteners

For single point failure fasteners, wire locking shall be used.

Rationale:	To avoid failure if single loosen fastener.

Note: Single point failure fastener is defined as an interface between two parts fixed with a single fastener.

- Fastener Lubricant

All stainless steel fasteners mating to flight inserts shall have a dry film lubricant Everlube 620C or equivalent coating (primer).

Rationale: 	Good design practice

- Self Tapping Screws

Self-tapping screws shall not be used.

Rationale: 	Good design practice

- Wet-mounted Units

All newly developed electronic units that will be wet mounted with Room Temperature Vulcanizer (RTV) to bus panels shall have a jacking feature or a jacking bolt to facilitate unit removal from the panel.

Rationale: 	To facilitate unit removal from the panel

- Release Agent

All electronic units that will be wet mounted with RTV to bus panels shall have a release agent applied as part of the wet mounting process to facilitate unit removal from the panel.

Rationale: 	To facilitate unit removal from the panel

Subsystem/Unit Vibration and Shock Hardware and Torque

The mounting hardware and torque requirements specification including that for the test harnesses connector mounting shall be in accordance with the applicable Interface Control Document (ICD).

Note: This requirement is applicable for low level resonance search testing, sinusoidal vibration testing, random vibration testing and shock testing.

Rationale: 	To ensure that the mounting hardware is compatible with the test equipment.

### Connector Requirements

- Connector Contacts

Connectors shall have gold plated contacts, as a minimum, per MIL-G-45204 (AD-12) Type II Grade C Class 1 and non-magnetic conductive metallic shell.

Rationale:	To ensure minimum gold plating quality for electrical contacts. To minimize magnetic moment and permeability, that can affect magnetometers and attitude control.

- Metallic Connector Shells

All external unit connectors shall include a metallic outer shell such that when the mating cable harness is inserted in the box mounted connector, the whole connector is electromagnetically shielded.

Rationale:	To maintain shielding integrity for the interconnected assembly.

- Connector Securing Screws.

All non-circular harness connectors, except those that are completely internal to a unit, shall be secured to the mating connector post using socket-headed cap screws.

Rationale:	Good design practice

- Connector Saver- Goal

Whenever possible, flight  connectors to be used for testing should be fitted with connector savers.

Rationale:	To minimize the number of connect/disconnect cycles.

- Non-Flight Connector Savers

Non-flight connector savers are permitted but shall have:

- gold plated, machined contacts;

- no prohibited materials as defined in AD-01, section 7.2.12;

- if planned to be used during Thermal Vacuum (TVAC) testing, comply with the outgassing requirements defined in AD-01.

Rationale:	To allow non-flight connector savers for cost savings while ensuring minimum quality to avoid damage to or contamination of the mating flight equipment.

- Temporary Connector Protective Covers

Temporary protective Electrostatic Discharge (ESD)-safe covers shall be applied to flight connectors.

Rationale:	To preclude entrance of contaminants into connector and to prevent damage from ESD during assembly, integration, test, shipping, handling and storage.

- Temporary Connector Protective Covers Removal Prior to TVAC

Temporary covers shall be removed prior to thermal vacuum testing.

Rationale:	To avoid possible outgassing issue of the protective cover material as well as to allow proper venting of the connector.

- Deleted

- Test Connectors Cover Plate

Test connectors shall be provided with a metallic blanking cover providing a continuous electrical shield over the connector for flight.

Rationale:	To protect from radiation at the interface and to protect the contacts from damage or false connection or debris.

- Connectors Requiring Termination Plugs

Connectors that require flight termination plugs containing electrical components or wire links, shall be accessible to install the flight plug after last use of the connector.

Rationale:	To minimize turnaround time at the launch site.

### Equipment Surfaces

- Equipment External Surfaces Treatment

All external surfaces (including structural elements but excluding alignment references) shall have a surface treatment to prevent corrosion and contamination due to outgassing.

Rationale:	To preserve integrity of the surfaces and to minimize contamination.

- Equipment Surfaces Treatment- Goal

The surface treatment used should be chosen from paint, chemical conversion coatings or non-metallic coatings as appropriate.

Rationale:	Common surface treatments.

### Electrical Bonding

- Bond Fault Current Rating

The bond shall have sufficient cross section to accommodate for an indefinite time, anomalous currents as per AD-20..

Rationale:	To ensure that the ground path can safely carry any ground fault current, with 50 % margin.

- Bond Straps Width to Length Ratio

- If bond straps are used, they shall be as per AD-20 sec 4.2.11  in order to minimize the inductance of the strap.

Rationale:	To minimize the inductance of the strap.

#### Electrical Equipment

- Electrical Equipment Bonding

All equipment having electrical connections (power and/or signal) shall be electrically bonded to the spacecraft structure.

Note: Good electrical bonding provides (RD-04):

- Fault current paths for protection against fire and personnel shock;

- A low impedance bond path for antenna installations to ensure no degradation of acceptable performance and to protect the antenna element from damage caused by P-static, radio frequency (RF) currents, or lightning events;

- A current path for RFs for proper operation of filters and shields;

- Protection against the effects of lightning;

- A means to prevent or safely discharge static charges.

Rationale:	To minimize electrical potential differences between equipment and individual parts of structure (RD-04).

- Electrical Equipment Bonding Impedances

All metallic parts of the structure shall be bonded together in accordance with bonding impedance values of Table 3-1.

Table 3-1 – Maximum Bonding Impedances

| Item # | Grounding / Bonding Connection | Bond DC Resistance Requirement |
| --- | --- | --- |
|  | Primary power return to structure (at single point in the power subsystem) | £ 2.5 mW |
|  | Between two mating metal structure parts | £ 2.5 mW |
|  | Between a moveable metal structure item and fixed structure | £ 2.5 mW |
|  | Unit case to structure | £ 2.5 mW |
|  | Unit or bracket mounted connector: between the Connector shell and the unit or bracket | £ 2.5 mW |
|  | Cable shield to harness connector backshell | £ 2.5 mW |
|  | Harness connector backshell to connector case | £ 2.5 mW |
|  | Units on Carbon Fibre Reinforced Polymer (CFRP) parts bonded to structure / ground rail (per junction) | £ 2.5 mW |
|  | Unit secondary power return to spacecraft structure | £ 10 mW |
|  | Active units: between any point on the unit and the mounting interface | £ 10 mW |
|  | Passive units: between any point on the unit and the mounting interface | £ 100 mW |
|  | Multi-Layer Insulation (MLI) bonding point to structure | £ 1 W |
|  | Between an aluminum facesheet and aluminum honeycomb | £ 10 W |
|  | Between a CFRP part and metal structure | £ 100 W |
|  | Between mating CFRP parts | £ 100 W |
|  | Between metal fittings on CFRP parts to the CFRP | £ 100 W |
|  | Between a CFRP facesheet and aluminum honeycomb | £ 100 W |
|  | Any other outer conductive components to structure | ≤ 1kΩ* |

- *In case 1 kOhm is not feasible, an appropriate value can be calculated based on section 6.3.1 g) of RD-03

Rationale:	To ensure good electrical contacts between surfaces and wherever available, with Direct Current (DC) resistance limits in accordance with RD-04.

#### Thermal Blankets

- Thermal Blanket Ground Points

Conductive layers of thermal blanket shall be bonded to each other and to the structure at a minimum of two (2) locations.

Note: If area of thermal blanket is <0.5m2, deviation may be accepted on a case by case basis if sufficient area is not there to install 2 bonding location.

Rationale:	To ensure that all layers are electrically connected together and redundantly connected to the structure.

- MLI Bonding Resistance

The resistance between any point of the metalized foils and the bonding point shall not exceed requirements per [WFS-GDIR-0340]  item r .

Rationale:	To ensure that there are no floating metalized foils. 1 kW is in accordance with requirement [WFS-GDIR-0340]  item r).

- MLI Minimum Number of Bond Points

The minimum number of bond points connections for MLI from bonding point to structure shall be as per Table 6-2.

Table 3-2 – Bond – Connections of Multi-Layer Insulation

| Size of blanket | No of bonds |
| --- | --- |
| <500 cm2 | 2† |
| < 1000 cm² | 2 |
| 1000 cm² - 40000 cm² | 3 |
| > 40000 cm² | Add one bond connection for each additional 40000 cm² |

Rationale:	In accordance with RD-04 and considering for minimum redundancy.

† Deviations may be accepted on a case by case basis for blankets not having sufficient area to install 2 grounds.

### Contamination and Control Requirements

- Provision of Protective Covers.

Protective covers to preclude entrance of foreign particles to sensitive areas, such as optical sensors, and to preclude damage during the handling, assembly, integration and test of the equipment shall be provided.

Rationale:	To preclude entrance of foreign particles to sensitive areas, such as optical sensors, and to preclude damage during the handling, assembly, integration and test of the equipment.

### Alignment

- Alignment Maintenance

The structure, equipment and their interfaces shall be designed such that alignment references can be maintained throughout bus and spacecraft integration.

Rationale:	To ensure that alignment reference targets are not obscured or removed during the spacecraft integration.

- Optical Targets

Optical reference targets shall be provided on all equipment where optical alignment techniques are to be employed.

Rationale:	To ensure that all equipment subject to optical alignment is equipped with the required calibration targets.

- Provision of Equipment References

Each equipment requiring alignment shall carry reference points, lines, or surfaces for use as alignment references.

Rationale:	To ensure that equipment subject to alignment with interface equipment is equipped with the required reference points, lines or surfaces.

- Alignment About Two Axes

Equipment requiring alignment or alignment measurement about two (2) axes shall carry three (3) points, two (2) lines or a surface that lie in a plane parallel to the plane containing those two axes.

Rationale:	Minimum condition options for alignment or alignment measurement about two (2) axes.

- Alignment About Three Axes

Equipment requiring alignment or alignment measurement about three (3) axes shall carry sufficient references to create two orthogonal planes.

Rationale:	Minimum condition options for alignment or alignment measurement about three (3) axes.

- Equipment Reference Stability

The hardware supplier shall allow for internal structure movements (e.g. bending and thermal distortion within the equipment) in the alignment budgeting.

Rationale:	Bending and thermal distortion effects will affect alignment and are to be considered in the alignment budget.

### Structural Design

- Design Safety Factors (SFd)

The Design Safety Factor values (SFd) defined in Table 3-3, Table 3-4 and Table 3-5 shall be applied to determine design loads for the given criteria.

Note: The safety factors in Table 3-3, Table 3-4 and Table 3-5 include the test tolerances.

Table 3-3 – Design Safety Factors – General

| Item | Criteria | SFd With a Test | SFd Without a Test |
| --- | --- | --- | --- |
|  | Metal Yield | 1.25 | 1.60 |
|  | Metal Ultimate | 1.40 | 2.00 |
|  | Critical Load (Buckling) | 1.40 | 2.00 |
|  | Structural Bonding And Inserts | 1.50 | 2.00 |
|  | Lifting Points And Fittings - Yield | 1.60 | N/A |
|  | Lifting Points And Fittings - Ultimate | 2.25 | N/A |
|  | MGSE – Metal Yield | 2.00 | N/A |
|  | MGSE – Metal Ultimate | 3.00 | N/A |
|  | Launch site MGSE – Metal Yield | 3.00 | N/A |
|  | Launch site MGSE – Metal Ultimate | 5.00 | N/A |
|  | MGSE - All part submitted to degradation (jacks, adjustment system,...) - Yield | 3.00 | N/A |
|  | MGSE - All part submitted to degradation (jacks, adjustment system,...) - Ultimate | 5.00 | N/A |
|  | MGSE - Lifting accessories (shackles, rings, etc.) - Yield | 3.00 See note (3) | N/A |
|  | MGSE - Lifting accessories (shackles, rings, etc.) - Ultimate | 5.00 See note (3) | N/A |
|  | MGSE - Lifting accessories (cables, chains, ropes, cables, slings, etc.) | See note (4) | N/A |

SFd without a test for Lifting equipment and MGSE is N/A as proof load testing is required.

For bolted joints, refer to [WFS-GDIR-0030]  .

Refer to [WFS-GDIR-0460]   .

Refer to [WFS-GDIR-0470]  .

Table 3-4 – Design Safety Factors – Sandwich Panels

| Item | Criteria | SFd With a Test | SFd Without a Test |
| --- | --- | --- | --- |
|  | Local collapse due to the shear stress on the honeycomb cell wall | 1.5 | 2.0 |
|  | Overall buckling on the cell wall (crimping) | 1.5 | 2.0 |
|  | Wrinkling of the skins | 1.5 | 2.0 |
|  | Intracell dimpling of the skins | 1.5 | 2.0 |
|  | Local crushing of the panel due to the insufficient compressive strength of the cell walls | 1.5 | 2.0 |
|  | Bond shear | 1.5 | 2.0 |
|  | Tension | 1.5 | 2.0 |

Table 3-5 – Design Safety Factors – Composites

| Item | Criteria | SFd With a Test | SFd Without a Test |
| --- | --- | --- | --- |
|  | Compression | 1.5 | 2.0 |
|  | Tension | 1.5 | 2.0 |
|  | Skin-core shear | 1.5 | 2.0 |
|  | Interlaminar shear | 1.5 | 2.0 |
|  | Shear for bonded parts | 1.5 | 2.0 |

- Additional design factors may be considered to account for any of the factors listed in Table 3-4.

For inserts, refer to AD-10.

- Rationale:	Common design practices.

- Lifting accessories: Shackles and Rings

- Lifting accessories like shackles, rings, etc. shall be delivered with a certificate of conformity, Conformity with European health, safety, and environmental standards (CE mark) or equivalent applicable regulatory marking.

- Rationale:	To ensure safe lifting of equipment. For acceptability in test lab and on launch site.

- Lifting Accessories: Cables, Chains, Ropes and Slings

- Lifting accessories like cables, chains, ropes, slings, etc. shall be delivered with a certificate of conformity and proof test, CE mark or equivalent applicable regulatory marking, and designed for the safe working load (1xSWL) with a safety coefficient of:

- Steel cables with ends: 5 ultimate limit;

- Lifting chains: 4 ultimate limit;

- Textile ropes or slings: 7 ultimate limit.

- Rationale:	To ensure safe lifting of equipment. For acceptability in test lab and on launch site.

- Additional Design factors

Additional design factors as listed in Table 3-6 should be applied to determine design loads for the given criteria.

Table 3-6 – Design Effects Requiring Additional Design Factors

| Item | Design Effects |
| --- | --- |
|  | Stress concentrations: Accounts for high local stress concentrations (e.g. at holes, corners or fillets). |
|  | Corrosion effects |
|  | Material: Accounts for property scatter, flans, brittle or fragile materials, variations in process control. |
|  | Creep |
|  | Welding/Casting: Accounts for process-control variations such as sensitivity to cooling rate and size. |
|  | Hazards: Accounts for safety of personnel and flight hardware when structure contains pressure or other stored energy. |
|  | Fittings: Accounts for unknown stresses in complex joints or fittings |

- Rationale:	To ensure that the designer accounts for all effects requiring imposition of additional design factors.

- Pressurized Equipment Design and Test

Propulsion elements and other pressurized equipment shall meet the design and test requirements of either:

- ANSI/AIAA S-080 “Space Systems – Metallic Pressure Vessels, Pressurized Structures and Pressure Components” (AD-17) and ANSI/AIAA S-081B “Space Systems – Composite Overwrapped Pressure Vessels (COPVs)” (AD-18), or

- ECSS-E-ST-32-02C Rev. 1 “Structural design and verification of pressurized hardware” (AD-15).

Note: Mixing of the above standards is forbidden.

- Rationale:	To assure the safety and to enhance the success of the operation of pressurized equipment.

- Fatigue Design

Metallic structures (including solder joints, printed circuit boards and component leads) subjected to a cyclical mechanical and thermal load condition, shall be designed for a fatigue life greater than the cumulative fatigue damage multiplied by the following fatigue safety factors:

- For design, a fatigue safety factor of four (4) for any element when material allowable used are obtained from empirical stress v.s. number of cycles curves.

- For design, a fatigue safety factor of two (2) for any element which has been validated by accelerated life tests.

- Rationale:	Safety factor of four (4) and (2) when respectively verified by analysis and test are appropriate for the uncertainty associated with the verification methods.

- Margin of Safety

Every element shall be designed to have a positive Margin of Safety (MS). The Margin of Safety is defined as:

MS = [Allowable design load (or stress)/Predicted design load (or stress)] - 1

Rationale:	To ensure that the predicted load (or stress) does not exceed the allowable load (or stress).

- Structural Welds on MGSE Load Path

Structural welds on MGSE load path(s) shall be avoided.

Rationale:	Common design practice.

### Mechanisms

In view of the criticality of space mechanisms, which are often potential mission critical single point failures, particular attention is placed upon the reliability and redundancy of space mechanisms (AD-11). The following requirements are selected from AD-11 and tailored or adapted as appropriate, based on mission class and needs.

#### Reliability and Redundancy

- Mechanism Failure Propagation

Failure of one part or element of mechanisms shall not result in consequential damage to spacecraft equipment.

Rationale:	Similar to ECSS-E-ST-33-01_0820015.

- Mechanism Minimum Redundancy

Unless redundancy is achieved by the provision of a complete redundant mechanism, active parts of mechanisms, such as sensors, motor windings, brushes, actuators, switches and electronics, shall be redundant.

Rationale:	Same as ECSS-E-ST-33-01_0820023.

- Mechanism Failure Affecting Redundant Element

Failure of one element or part of the mechanism shall not prevent the other redundant element or part from performing its intended function, nor the mechanism from meeting its performance requirements.

Rationale:	Similar to ECSS-E-ST-33-01_0820024.

#### Function

- Mechanism Specifications and Verification

Mechanisms mechanical interface, position accuracy or velocity tolerances shall be specified and verified that they conform to the functional needs.

Rationale:	Same as ECSS-E-ST-33-01_0820030.

- Mechanism Envelope of Movement Definition and Verification

The envelope of movement for each moving part of the mechanisms shall be defined on the ICD and verified by inspection.

Rationale:	Similar to ECSS-E-ST-33-01_0820031.

- Mechanism No Mechanical Interference

The movement of each mechanism part shall ensure that there is no mechanical interference with any other part of the mechanism, the spacecraft, the payload or the launcher.

Rationale:	Same as ECSS-E-ST-33-01_0820032.

#### Operational Constraints

- Mechanism No Operational Constraints- Goal

The mechanism should not impose any operational constraints on the spacecraft and mission.

Rationale:	Same as ECSS-E-ST-33-01_0820455.

#### Tribology

- Mechanism Sliding Surfaces

The use of sliding surfaces on mechanisms shall be avoided, unless one of the sliding surfaces is hard and the other one lubricated or composed of a self-lubricating material.

Note: Example of self-lubricating material: polyimide resins.

Rationale:	Similar to ECSS-E-ST-33-01_0820061 and ECSS-E-ST-33-01_0820062.

- Mechanism Sliding Contacts Materials

Metal to metal tribological sliding contacts of mechanisms shall be composed of dissimilar materials as defined in AD-01.

Rationale:	Similar to ECSS-E-ST-33-01_0820457.

- Mechanism Dry Lubrication Tribological Surfaces Samples

During the lubrication of mechanism tribological surfaces, samples of representative material, surface roughness, surface cleanliness and surface orientation shall be co-deposited in each process run with the flight components so that thickness and adhesion of the lubricant on samples can be verified.

Rationale:	Similar to ECSS-E-ST-33-01_0820068.

- Mechanism Fluid Lubrication Environmental Effects

The effect of exposure to on-ground storage and related gravity effects, and other ground or in-orbit accelerations on mechanisms lubricant distribution shall be validated.

Rationale:	Same as ECSS-E-ST-33-01_0820073.

- Mechanism Fluid Lubrication Anti-creep Barriers

Provisions shall be included to avoid migration of fluid lubricants to the external sensitive equipment defined by the customer.

Note: It is also important to use the anti-creep barriers for sensitive equipment within the mechanism.

Rationale:	Same as ECSS-E-ST-33-01_0820076.

- Bearing Pre-Loading.

Ball bearings in mechanisms or other spacecraft equipment shall be pre-loaded with a load calculated in order to withstand the mechanical environment during launch and throughout the mission.

Rationale:	Similar to ECSS-E-ST-33-01_0820080.

#### Mechanical Design

- Mechanism Specific Factors of Safety

The following specific factors of safety shall apply for mechanisms:

- For cables, stress factor of safety against rupture: 3

- For stops, shaft shoulders and recesses, against yield: 2

Note: For other applicable factors, refer to [WFS-GDIR-0450]  .

Rationale:	Similar to ECSS-E-ST-33-01_0820117.

- Mechanism Dimensioning

Mechanisms shall meet the functional dimensioning requirements of ECSS-E-ST-33-01 section 4.7.5.3 (AD-11).

Rationale:	As per ECSS-E-ST-33-01.

- Mechanism Status Monitoring

Unless monitored at spacecraft level, the design of mechanisms shall include means to monitor the execution of its main functions.

Rationale:	Same as ECSS-E-ST-33-01_0820135.

- Mechanism Status Monitoring of Critical Functions

Mission critical mechanisms shall be designed in such way that monitoring information of its critical functions is part of the spacecraft telemetry.

Rationale:	Same as ECSS-E-ST-33-01_0820136.

- Mechanism Latching and Locking

Mechanisms shall meet the latching and locking requirements of ECSS-E-ST-33-01 section 4.7.5.4.3 (AD-11).

Rationale:	To avoid inadvertent opening of the mechanism due vibration or shock.

- Mechanism End Stops

Mechanisms with restricted travel or rotation shall be provided with regular or emergency mechanical end stops to limit their motion and travel extremes.

Rationale:	Similar to ECSS-E-ST-33-01_0820146.

#### Electrical Design

- Mechanism Electrical Wires Insulation

Mechanism electrical wires shall be insulated from the mechanism structure and from each other by not less than 100 MΩ measured with a DC voltage of 500 V applied.

Rationale:	Similar to ECSS-E-ST-33-01_0820193.

- Mechanism Motor Windings Insulation

Mechanism electric motor windings shall be insulated from the mechanism structure and from each other by not less than 100 MΩ measured with a DC voltage five times the worst-case flight operating voltage.

Rationale:	Similar to ECSS-E-ST-33-01_0820194.

- Mechanism Overcurrent Protection

Mechanisms containing electrical parts and circuitry shall be protected against overcurrent.

Rationale:	Similar to ECSS-E-ST-33-01_0820207.

- Mechanism Control System Gain and Phase Margin

Mechanisms control systems shall meet the gain and phase margin requirements of ECSS-E-ST-33-01 section 4.7.8.1 and 4.7.8.2 (AD-11).

Rationale:	To ensure robustness and reliability of the mechanism control loop.

- Mechanism Harnesses and Cables to Moving Parts

Harnesses and cables to moving parts shall be:

- characterized in terms of hysteresis and stiffness in representative configuration over the full range of displacement and over the specified qualification levels in terms of temperature range, lifetime, speed effects;

- taken into account in the control system design.

Rationale:	Similar to ECSS-E-ST-33-01_0820228 and ECSS-E-ST-33-01_0820229.

- Mechanism Analysis of Control Systems

Mechanisms control systems shall be analyzed as per the requirements of ECSS-E-ST-33-01 section 4.8.2.13 (AD-11).

Rationale:	To ensure that the mechanism function, performance and robustness is properly analyzed.

#### Verification

- Mechanism Verification by Test

Mechanisms shall meet the verification by test requirements of ECSS-E-ST-33-01 section 4.8.3 (AD-11), with the exception of the following paragraphs or clauses:

- 4.8.3.4.2.d: Replaced by: “After acceptance testing, refurbishment shall not be performed.”;

- 4.8.3.4.4: Acceptance test criteria to be in line with mechanisms requirements specification.

Rationale:	For item a), refurbishment should not be baselined in the design and avoided after acceptance test. For item b), applicable criteria are as per specific mechanism requirements specification and will be defined in the test plan.

### Material Properties and Allowables

- Test Data For Composites and Potted Inserts

Strength of material values, used in the design process, shall be validated by coupon testing, and verified by end product testing, as follows:

- Composite raw material properties to be verified by supplier lot/batch testing during material manufacture.

- Composite panel (sandwich) properties - verification by work-in process testing of representative sample(s) during manufacture of flight panels.

- Potted inserts:

  - Design allowables and manufacturing processes validated by development coupon testing.

  - Where applicable, polymeric hardness testing of potting compound(s) sample(s) taken during manufacture of flight panels

  - Flight insert workmanship and installation processes verified by testing of representative sample(s) from flight panels.

Rationale:	To ensure appropriate validation for material properties of composite material and inserts.

### Venting and Depressurization

- Depressurisation Venting

Venting, to allow for depressurization, shall be included in the design of the spacecraft equipment.

Rationale:	To ensure appropriate and predictable depressurization rate.

- Direction of Venting

No vent hole shall be in the line-of-sight of any optical instrument.

Rationale:	To avoid contamination of the optical surfaces from the venting products.

## Thermal Design and Interfaces

### General

- Temperature Extremes

The equipment shall be designed to withstand the temperature extremes which may be experienced during transportation, testing, launch and orbital life, as described in EDTS Sections 4.2.3, 4.2.4, 4.2.5 (AD-19).

Rationale:	To ensure that design considers for worst-case temperature extremes.

### Thermal Hardware

- Maximum Duty Cycle of Heaters

The maximum duty cycle of heaters shall be 70% under worst-case cold environmental
conditions when heaters are implemented in a thermal control design.

Rationale:	As per  AD-05, section 3.4.2.2

### Temperature Control

- Temperature Control – Thermal Subsystem

The spacecraft thermal control shall maintain on-orbit spacecraft temperature profiles within the applicable subsystem/unit level specifications in all spacecraft operational and survival modes.

Rationale:	To ensure that the thermal control subsystem will maintain the equipment within its temperature limits, under worst-case spacecraft modes and conditions.

- Test Temperature Sensors Locations

The locations of the test temperature sensors shall be selected in accordance with the Contractor assessment and be accessible for installation of temperature sensors.

Note: These locations will be used to monitor temperature during thermal testing.

Rationale:	To ensure that components and/or critical parts of the unit or spacecraft achieve the desired temperature for the required time during thermal testing.

- Temperature Sensors Unit Location

The temperature sensors shall be installed by the contractor on  the exterior surface of the unit (Temperature Reference Points (TRPs)), in addition to component base plate or heat sink, if temperature sensing requirements are specified for these mounting interfaces, and instrumented for use during spacecraft and unit thermal and TVAC testing.

Rationale:	These TRPs allows to monitor the temperature of critical instrument components during testing and flight.

- Test Temperatures

Test temperatures for thermal and thermal vacuum soak shall be based on temperature sensors installed at TRPs.

Rationale:	Common practice.

- Temperature Sensors Access

The locations of non-flight temperature sensors shall be readily accessible to enable disabling and securing them for flight.

Note:  It is anticipated that the temperature sensor at the Temperature Reference Point will remain operative for flight to gather thermal data in support of on-orbit thermal analysis and enhancement of the thermal math model.

Rationale:	Intent is to mitigate undue disturbances to the flight harnesses and minimize contact with spacecraft equipment.

### Thermal Interface

- Paint Free Areas

Equipment shall have paint free areas for base contact, mounting of heaters and electrical bonding resistance measurements.

Rationale:	To promote good thermal and electrical contact.

- Thermal Filler

For thermally coupled units, the contact area between conductive thermal sinks and units shall be compatible with the use of a thermal filler.

Rationale:	To promote good thermal contact and mitigate material degradation.

- Temperature Margins

The minimum temperature margins shown in Figure 6-1 and described as follows shall apply:

- Predicted Temperature margin of 5 ºC above the maximum worst-case predicted temperature and 5 ºC below the minimum worst-case predicted temperature (leading to the Allowable Flight Temperature Range);

Note: Heater controlled components are exempted from this requirement for the cold side of the temperature range if the heater duty cycle does not exceed 70%.

Flight Acceptance margin of 5 ºC above the maximum Allowable Flight Temperature and 5  ºC below the minimum Allowable Flight Temperature (leading to Flight Acceptance Temperature Range);

Proto-flight (or Qualification) margin of 5 ºC above the maximum Flight Acceptance Temperature and 5 ºC below the minimum Flight Acceptance Temperature (leading to Proto-flight (or Qualification) Test Temperature Range).

Figure 3-1 – Temperature Margins (Reference AD-05)

Rationale:	Standard modelling uncertainty, acceptance and design margins, as extracted from AD-05.

## Electrical Design and Interfaces

### Test Stimuli and Test Points Interfaces

- Internal Connectors Connection

The spacecraft internal connectors shall not be disconnected after completion of the spacecraft acceptance test program.

Rationale:	To ensure that the hardware configuration is not perturbed after formal acceptance.

- Test Points

Test points shall be provided to enable monitoring and stimulation of key functions from the exterior of the spacecraft.

Rationale:	To ensure that complete test and calibration of an equipment can be executed at spacecraft level without physical access to the equipment inside the spacecraft.

- Telemetry Monitoring During Tests

The spacecraft test connectors shall allow all telemetry to be monitored, without the necessity of activating Radio Frequency (RF) transmitters.

Rationale:	To avoid unnecessary exposure of the test personnel to RF radiation, to minimize usage of the transmitters and to allow testing of individual subsystems without the use of transmitters.

- Test Point Locations

All test points that are to be accessible at Bus and Payload levels of assembly shall be grouped on dedicated connectors either on the units or in the harnessing such that they can be accessed without disconnecting flight connectors.

Rationale:	To ensure that complete test and calibration of an equipment can be executed at Bus or Payload level without physical access to the equipment inside the Bus or Payload and without having to disconnect other connectors.

- Test Point Faults

Test points and test stimuli on multi-pin connectors shall be designed to withstand, without causing damage:

- voltages up to the highest voltage on that connector;

shorts to ground;

shorts line to line;

maximum fault voltage from EGSE.

- Rationale:	To avoid hardware under test electrical overstress (EOS) or failure due to test operator error or test software bug.

- Automated Flying Probe

Flight printed wiring board assembly testing shall not use an automated Flying Probe process unless approved by the CSA.

Rationale:	To ensure that appropriate test points are implemented in the printed wiring board design and to avoid mechanical damages or electrical overstress induced by flying probe testing.

### Power Stability

- Power Stability

Equipment power converters shall be stable under worst case conditions.

Rationale:	To ensure that power converters provide stable output voltage given any load or environmental condition and when considering for ageing of the system.

### Switch OFF Characteristics

- Equipment On/Off Control

All spacecraft units shall have the capability to be powered on and off by ground command, save for the essential loads (e.g. S-band receiver, PCU/PCDU, C&DH).

Rationale:	On and off commanding capability for non-essential loads is required for limiting power usage, ageing and to react to nominal and off-nominal situations. However, the essential loads need to remain on all the time and switch off capability is not desired to avoid the possibility of a “dead mode” situation.

- Power Cycling of Essential Loads

Power cycling of essential loads shall only be done via ground command.

Rationale:	To avoid possibility of a "dead mode" situation.

- Power Cycling of Essential Loads OFF Condition

During power cycling of essential loads, the OFF condition shall be strickly temporary.

Rationale:	To avoid possibility of a "dead mode" situation.

- Equipment Off Leakage Current

For units with ON/OFF switching within the unit, when commanded OFF, the leakage current drawn by the “OFF” unit shall not exceed 200 µA.

Note: The value of leakage current can be tailored if necessary.

Rationale:	To limit the power drawn by the off unit to few milliwatts.

- Switch Off Voltage Transient

Equipment switch off voltage transient superimposed on the main bus impedance shall not exceed allowable bus voltage limits.

Rationale:	To avoid unacceptable transients during unit switch off that could affect other units.

### Switch ON Characteristics

- Inrush Current

Switch current limit shall not be exceeded by charge transfer during unit power on.

Rationale:	To ensure that the switch is not tripped off due to inrush current, for any given condition.

- Charge Transfer

Charge transferred during power on shall not exceed 50 % of the switch’s maximum allowable charge transfer specification.

Note: For example, if a switch has a limit of 1 A*ms without tripping off, the maximum charge transfer during power on will not exceed 0.5 A*ms.

Rationale:	To ensure two times margin for inrush charge v.s. switch trip-off characteristic.

### Operational Transients

- Peak Transient Power

Unit peak power shall not be exceeded during operational power transient.

Rationale:	To ensure that the specified unit peak power is in fact the maximum worst-case peak power.

- Operational Transient

Conducted voltage transients on the primary power bus, appearing during nominal mode switch (excluding On/Off) shall not trigger load shed when operating at lowest nominal bus voltage.

Rationale:	To ensure that momentary (transient duration) undervoltage event on the bus (for example caused by another unit inrush current or operational transient) does not trigger an involuntary load shed action.

### Protection

- No Fuses for Spacecraft Equipement

Un-resettable fuses shall not be used in any spacecraft equipment.

Note: Resettable fuses protection is recommended.

Rationale:	To avoid rework when on ground and to provide attempt reset function once on orbit.

- Reverse Polarity

Each spacecraft unit shall include protection against accidental application of power with reverse polarity voltage.

Note:  A reverse polarity shunt diode is acceptable as a protection since the power switches to the payload are required to be current limited.

Rationale:	To prevent damage that could be caused by operator error.

- Command and Telemetry Circuits Fault Tolerance

Command and telemetry circuits (either analog, digital, pulse or serial) shall be capable of withstanding a short circuit to ground on each line, or a short circuit in between lines or from line-to-line, or open circuit, for indefinite time without damage or degradation after short or open condition removal.

- Rationale: 	To avoid rework due to incorrect wiring during tests or integration on the spacecraft.

- Command and Telemetry Circuits Fault Interference

A short circuit to ground or line-to-line or an open cicuit of a command or telemetry circuit (either analog, digital, pulse or serial)  shall not affect the operation or electrical characteristics of the other command and telemetry circuits.

- Rationale:	To avoid a fault on one command or telemetry circuit affecting the other circuits.

- No Exposed Electrical Conductors Inside the Spacecraft ssembly

There shall be no exposed electrical conductor inside the spacecraft assembly.

- Rationale:	A metallic Foreign Object Debris (FOD) floating freely in zero gravity inside the spacecraft should not be able to cause any short circuit.

- Ground Commanded Turn On.

In the event of any automatic load shed action or trip-off, the affected unit shall only be turned back on by ground command.

Rationale: 	Analysis of the issue needs to be carried out before turning the unit safely back on.

- Ground Commanded Reset.

Any autonomous switch off action shall provide the capability of being reset by ground command.

Rationale: 	Analysis of the issue needs to be carried out before turning the unit safely back on.

- Load Shed Generation

The spacecraft shall automatically generate a load shed (e.g. automatically power down payload and non- essential bus equipment) whenever non expected power conditions are detected that may endanger the spacecraft.

Note: Load sheds are generated in a controlled sequence when the battery state of charge drops below predefined levels. The spacecraft progressively reduces power consumption by powering off units, and may change operating modes, as the state of charge of the battery declines. Normal operational transient conditions are expected power conditions and should not result in a load shed

Rationale: 	To ensure spacecraft integrity in case of anomalous conditions and to reduce power consumption to a minimum safe level under conditions hazardous to the health or safety of the spacecraft.

- Load Shed Function Capabilities

The load shedding function shall be reversible via ground command.

Rationale: 	To ensure spacecraft has recovered before reversing the load shed.

- Communication Link Idle Watchdog

The spacecraft shall autonomously enter safe-hold mode if no commands are received from the ground for seven days.

Note: It is here assumed that, after seven days without receiving commands, there is a problem with the spacecraft receiving functions.

Not needed if the spacecraft has hardware decoded command or other means to force a reset of the command receive function. The intent here is to have a way to get out of a command receiving function bug, i.e. have a reset function.

Rationale: 	To attempt restoring the communication.

Keyed connectors

Keyed connectors shall be used where same type of interface connector is used on a unit with same pin configuration. Ex Prime and redundant.

Rationale:	To prevent connector misdating.

### Voltage Tolerance

- Undervoltage Range

All spacecraft units shall survive an under-voltage condition down to 0 V on the main Power Bus, with no damage or performance degradation after normal voltage is restored.

Note: Equipment may be designed to automatically switch off in order to survive an undervoltage.

Rationale:	To ensure that all units are capable of withstanding their own outrush/inrush current conditions caused by worst-case undervoltage conditions (amplitude and duration).

- Overvoltage Range

All equipment shall survive a permanent overvoltage condition on the main Power Bus, with no impact on specified performance, reliability or life after normal voltage is returned.

Note: Payload and non-essential Spacecraft Bus equipment may be designed to automatically switch off in order to survive a steady-state overvoltage above their upper operating voltage limit.

Rationale: 	To ensure survival of equipment under overvoltage condition.

### Power

- Battery Discharge in Cold Launch Configuration

While in a cold launch configuration, the batteries shall be able to hold a charge for fourteen (14) days with a resulting depth of discharge of no more than 20 %.

Rationale: 	To minimize risk against a delayed launch.

- Battery Usage During Eclipse

The batteries shall be sized to be able to provide all electrical power during operation in eclipse as well as during operations that may orient the solar panels away from the sun.

Rationale: 	To be able to operate in eclipse.

- Battery Usage During Deployment

The batteries shall also provide the power during the deployment phase (if applicable), from spacecraft separation form launch vehicle until sun acquisition by the solar array(s).

Note: The batteries may also be used to augment the solar array(s) during high peak loads.

Rationale: 	To be able to operate before and during deployment.

- Battery Power for Fault Clearing

The batteries shall be able to provide any transient power required to sustain tripoff current and transient loads.

Rationale: 	To maintain bus voltage during fault events to prevent load shed.

- Battery Aging Prior to Launch

Degradation due to batteries charge and discharge cycles prior to launch shall ensure battery degradation does not violate the mission requirements.

Rationale: 	To ensure battery degradation does not violate the mission requirements.

- Single Battery Cell or String Failure Tolerance

The spacecraft shall remain fully operational in the event of failure of any single battery cell.

Rationale: 	The Contractor is encouraged to evaluate redundancy schemes for all critical systems in order to remove single point failures. Depending on the configuration of the battery, the failure of one cell could result in the loss of a complete battery string. Therefore, one should make sure that, in the event of a single-cell failure, the battery remains with a sufficient capacity to maintain the minimum required spacecraft energy storage capacity.

- Solar Array Single String Failure Tolerance

The spacecraft shall remain fully operational in the event of failure of a solar array single string failure.

Rationale: 	To ensure that solar array provide sufficient power generation after failure of solar cell, string failure due to MMOD, etc. The Contractor is encouraged to evaluate redundancy schemes for all critical systems in order to remove single point failures..

Power Provision from Solar Array Simulator

During environmental and functional tests the spacecraft shall be capable of receiving power from a Solar Array Simulator via a spacecraft umbilical connector.

Rationale: 	To verify functionality and performance of the spacecraft when powered from solar array or simulated solar array. The solar array power conditioner may be noisy and may affect camera imaging reading quality and other sensitive equipment.

- Power Requirement

The spacecraft shall provide sufficient electrical power required to ensure that the spacecraft meets all performance requirements during all phases of the mission with applicable margin.

Note: The spacecraft needs to stay power positive at all times, including, but not limited to, Launch and Early Orbit Phase (LEOP) and safehold mode.

Rationale: 	To ensure that the spacecraft power subsystem is sized to supply the maximum aggregate load current, considering the worst-case beginning and end of life conditions and the applicable design maturity margin. Battery capacity and maximum depth of discharge are factors to be considered.

### Part Stress and Derating

- Part Stress Analysis

The parts stress analysis shall be performed concurrently with the packaging and thermal design in accordance with AD-01. Rationale: 	To ensure that the packaging and thermal control designs are done concurrently with the parts stress analysis and that the worse case stresses resulting from worst case environmental and operational scenarios are used in the analysis.

## TT&C related requirements

### General Requirements

- Independent Commanding

If a command causes the execution of more than one action, these actions shall be strictly operationaly interrelated.

Rationale: To prevent the execution of an unwanted action.

Execution Failure

The failure to execute a particular command of a pre-programmed sequence shall not result in permanent damage or degradation to any on board equipment.

Rationale: To prevent damage to the spacecraft due to an incorrect command sequence.

Telemetry from Direct Measurements

The telemetry information shall be provided from direct, unambiguous measurements rather than from secondary effects.

Rationale: To avoid having to derive status information from multiple secondary effects, which could lead to errors, especially in abnormal conditions.

Status without Telecommand History

The complete status of the spacecraft shall be derivable from the telemetry (TM) without the need for reference to the telecommand history.

Rationale: To provide sufficient telemetry such that the effects of commands, such as mode changes, are observable in telemetry.

- Toggle Command

Any "toggle" type of commands, that changes (or toggles) a state based on the current state without any explicit arguments shall be prohibited.

Rationale: 	To avoid having to predict the state at the moment of execution.

- Self-Contained Telemetry Packets

The value of telemetered parameters shall be self-contained (i.e. do not contain delta changes since last telemetry).

Rationale:	To prevent ambiguity in telemetry interpretation and avoid having to rely on past telemetry.

- Command Status

The spacecraft shall provide under all circumstances, positive and unambiguous status information for all commands that have been received, executed and rejected.

Rationale:	Common design practice. To ensure constant knowledge of command status.

- Functional During Anomalies

The command function shall remain functional during spacecraft abnormal or emergency conditions or situations.

Rationale: 	To ensure that the spacecraft can be commanded during diagnostic and recovery procedures.

- Enablig and Disabling Fault Detection Functions

The command function shall allow resetting of the fault detection software functions, enabling and disabling each fault-detection software function by ground control.

Rationale: 	To maintain the capability to operate the spacecraft even during anomalies.

- Use of No-Ops Command

An ineffective non-operating command shall exist to test the communication link.

Rationale: 	Common design practice.

- Command Function Stability

The spacecraft command function shall not change state or generate spurious commands, including when Bus power transients are present under normal operations.

Rationale: 	Common design practice. To minimize risks.

- Receive and Respond to Commands

The command function shall receive and respond to commands whenever power is supplied to the spacecraft Bus.

Rationale: 	To ensure the spacecraft is always commandable when powered.

- Emergency Engineering Telemetry

The flight system design shall have at least one configurable (data points and rates) engineering emergency telemetry for diagnostic use.

Rationale:	To diagnose spacecraft anomalies via telemetry.

- Hazardous Commands Activation and Enabling

All hazardous command (e.g. pyros, propulsion, or any command that can endanger the mission, the s/c safety, instrument safety, personnel safety, etc.) shall first require an enabling via separate command before activation of the command.

Rationale:	To guard against inadvertent activation of hazardous functions, no single command can result in commanding an hazardous function and need to result from at least 2 commands.

- Hazardous Command Enable/Disable

The flight software shall report the enable/disable state of all hazardous commands in the telemetry.

Rationale:	To know the status of any hazardous function.

- Assessment of Spacecraft State

The spacecraft telemetry design shall permit the operation center to unambiguously assess the spacecraft state during the early part of any ground contact pass, in particular to detect if the spacecraft is in a fault protection mode.

Rationale:	For the ground system to quickly initiate adequate commanding sequences consistent with the current spacecraft state.

- Invalid/Incorrect/out of Context Command Checking

The flight software shall be designed to provide protection against invalid and incorrect commands, and to respond safely under such conditions.

Rationale:	To guard against execution of invalid commands.

## Software Design and Interfaces

- Software Version in Telemetry

All flight software (spacecraft and instrument) shall report the unit ID and software version in the telemetry data so as to uniquely identify the unit and configuration that generated the telemetry.

Rationale:	To uniquely identify the unit and configuration that generated the telemetry.

- Firmware Version in Telemetry

For all firmware (spacecraft and instrument) it shall be possible to uniquely determine from telemetry its firmware version, whether directly (i.e. the device’s firmware generates telemetry containing the firmware version) or indirectly (e.g. another device determines this device’s firmware version by getting and reporting the firmware executable image’s checksum).

Rationale:	To uniquely identify the unit and configuration that generated the telemetry.

- Software and Firmware Protection from Unintended Modifications

Flight software and firmware that are modifiable during flight shall be protected from unintended modifications including those caused by operation errors, single event effects, and hardware problems.

Notes: Protection is typically provided by intentionally enabling a write operation before modifying the software; at all other times write operations are disabled to protect the software/firmware from unintended modifications.

The requirement does not prevent on-board memory scrubbing.

Rationale:	To prevent unintended modifications of the software and firmware.

- EDAC for On-Board Memory

All processor memories, standalone memories (volatile and non-volatile) and science data storage   should be equipped and operated with error detection and correction (EDAC), except scientific data storage.

Rationale:	Best practice.

- Multiple Flight Software or Firmware Images

When reprogrammable non-volatile memory is used to store flight software executable or firmware, three (3) or more copies of each flight software executable or firmware image shall be stored.

Rationale:	Multiple copies to overcome corruption of executables by on-orbit environment.

- Minimalist Boot

The bus should include a minimal version of the flight software that excludes tasks not essential to supporting on-board software maintenance (e.g. software update) and for which all exceptions or timeouts that could cause a reset are disabled.

Rationale:	To facilitate recovery and maintenance.

- Boot Process Verification

Upon failure to boot from a given flight software image, the bootloader shall iterate through other flight software images.

Note: Typically a Cyclic Redundancy Check (CRC) verification of the flight software image is done prior to attempting the boot, and a watchdog timer is used to ensure the successful outcome of the boot process.

Rationale:	To ensure the spacecraft boots in a known state.

- Flight Software Boot Counter

The flight system shall report in telemetry a boot counter incremented upon each flight software reboot.

Rationale:	To keep track of software reboot.

- Internal clock-related Counters and Timers

Design of spacecraft clock-related counters and timers shall be sized (both on hardware registers and software variables) to meet at least 2 times the extended mission life.

Note: This means that no spacecraft or instrument clock, nor critical counters, shall repeat itself (wrap around) during the prime mission.

Rationale:	To avoid time counter rollover during mission.

- Software Upload Capability

All software and data/configuration tables shall be uploadable from ground.

Rationale:	To maintain the spacecraft throughout the mission lifetime.

- Firmware Upload Capability

All reprogrammable FPGAs firmware shall be uploadable from ground.

Rationale:	To maintain the spacecraft throughout the mission lifetime.

- Spacecraft Clock in Telemetry

Spacecraft clock and instrument clock shall be reported in the telemetry to monitor clock drift rates with respect to ground clocks.

Rationale:	To monitor clock drift rates with respect to ground clocks.

- Real-time Failures and Watchdog Timers

Software systems (spacecraft and payload) shall be designed to detect failure to complete tasks within allotted time and to react appropriately.

Note: External watchdog timers are well suited to provide such capability. Critical tasks would reset a watchdog timer upon completion. Failure to reset the watchdog timer on time would result in a fault protection response appropriate to restore the system/software.

Rationale:	To protect against SEUs and other harmful events that could compromise real-time critical processes on-board.

- Software Initialization

The nominal operational flight software shall be designed to initialize all software and hardware elements to known, safe and defined state.

Rationale:	To ensure that the spacecraft initializes in a known and safe state in nominal operations.

## Wiring Harness Design and Interfaces

- Deleted

- Electrical Functions Classes Separation

For spacecraft harnesses, the signals and circuits from each of the different classes of electrical functions should be routed on distinct connectors/cables and physically separated on different bundles.

Note: When goal cannot be met, the signals and circuits from each of the different classes should be separated in the connector by a set of unused pin locations. Particular attention should be paid to the pyrotechnic signals. The classes of electrical functions are:

- Class 1: Primary and Secondary Power signals

- Class 2: Digital, Discrete and High-level Analog signals

- Class 3: Pyrotechnics signals

- Class 4: Low-level Analog signals (potentially susceptible to EMI)

- Class 5: RF signals

Rationale:	To minimize signals crosstalk and EMI.

- Twisted Pairs

Wires within a pair shall be twisted.

Note: It is recommended to avoid single-ended interfaces.  In the case where single-ended signals are used, it is recommended to use single wires bundled together and covered by a common shield.

Rationale:	To minimize the coupling loop area and therefore reduce magnetic pick-up and emission.

- Differential Signal Distribution

Differential signals shall use shielded twisted pair wire.

Rationale:	To minimize noise and voltage offsets.

- Twisted Bundle

When several lines share a return line, the whole bundle shall be twisted with its return wire.

Rationale:	To minimize the coupling loop area and therefore reduce magnetic pick-up and emission.

- Twisted Pairs Routing on Connectors

Twisted-pair wires shall be routed through connectors on adjacent pins.

Rationale:	To minimize the coupling loop area and therefore reduce magnetic pick-up and emission.

- Connector Backshells

All cables shall use RF backshells on the connectors.

Rationale:	To maintain electromagnetic shielding integrity.

- 90 degree End Entry Backshells

End entry (90°) connector backshells where the mounting hardware or jack posts run through the inside of the backshell shall be avoided.

Rationale:	In end entry (90 º) backshells, one of the 2 jackscrew could interfere with the wiring inside and damage insulation.

- Overall Shield Termination

The overall shields on cable bundles shall be terminated on the backshells of connectors in a 360° shield termination.

Rationale:	To maintain electromagnetic shielding integrity.

- Shield Tails

Where an individual shield is passed through a connector, the shield 'tail' length shall not exceed 5 cm.

Rationale:	To minimize parasitic impedance, with maximum length compatible with the workmanship standards. Minimal length is preferable.

- External Cables

Cables and wires that are mounted on the external part of the spacecraft structure shall have overall braided shields covering the external sections, with the shields terminated on the outside of the spacecraft structure (excluding solar panels provided that Polytetrafluoroethylene (PTFE) coating or insulations are not used) at the points of entry/exit to the structure.

Rationale:	To maintain electromagnetic shielding integrity and to provide additional protection against Micrometeroids and Orbital Debris (MMOD).

- Coaxial RF Cables Test

Coaxial RF Cables shall be tested post installation to ensure insertion and return loss are within acceptable values.

Rationale:	To ensure that the installation and mounting tension/deformation does not affect RF performance.

- Magnetic Moment

The design and routing of wiring harnesses shall consider magnetic moment.

Rationale:	To minimize possible interference with magnetometer and attitude control.

## Launch Vehicle Interface

- Spacecraft Interfaces to Launch Vehicle

The spacecraft shall provide the mechanical and electrical interfaces with the selected launch vehicle.

Rationale: 	To ensure proper mechanical attachment of the spacecraft to the launch vehicle and to allow communication with the spacecraft once integrated into the launch vehicle.

- Spacecraft Umbilical Connector

The spacecraft shall incorporate an umbilical connector directly accessible at the exterior of the spacecraft.

Note: Basic functional verification incudes connection of a solar array simulator for end-to-end verification of the power subsystem, battery charging, interface to on-board computer, etc

Rationale: 	To allow battery charging and basic functional verification of the spacecraft during ground testing and pre-launch operations

- Spacecraft Umbilical Connector Launch Vehicle Signals

The signal complement on the spacecraft umbilical connector shall be as defined by the Launch Vehicle and launch range interface, plus any additional satellite-specific signals.

Note: Details of umbilical connection will comply with the selected Launch Vehicle.

Rationale: 	To ensure that all required signals from both launch vehicle and spacecraft are available on the umbilical connector.

- Spacecraft Maximum Volume Envelope

The maximum envelope of the stowed spacecraft shall be within the volume requirements of the selected launch vehicle, and including applicable static and dynamic envelope margins.

Rationale: 	To ensure that the spacecraft will fit within launch vehicle and with sufficient clearance to the fairing.

- Spacecraft Mass

The total wet mass (mass with fuel) of the stowed spacecraft, including applicable margin, shall not exceed the mass limitations imposed by launch vehicle requirements.

Note:  The contractor will demonstrate that the budgeted mass does not exceed the launch vehicle requirements.  Should the contractor opt to use a tug to transport the spacecraft from launch vehicle separation to operational orbit, the spacecraft budgeted mass would be required to meet the tug’s mass constraint.

Rationale: 	To ensure that the spacecraft wet mass does not exceed launch vehicle capability to put the spacecraft into the desired orbit.

- Cold Launch Configuration

The spacecraft shall support a cold launch configuration.

Note: The cold launch configuration keeps the satellite electronics off until released from the launch vehicle.

Rationale: 	To minimize risk of accidental deployment, maximize the number of launchers compatible with the spacecraft, minimize battery depletion and remove the spacecraft design constraint of having to be capable of operating under partial pressure.

## Electrostatic Discharge Control

- Surface Charging

All materials used on the external surface of the spacecraft shall have surface resistivity of ≤ 109 ohms, over temperature, life, vacuum and mechanical stress conditions, and be grounded to the spacecraft structure.

Rationale:	As per section 5.2.1.3.1 of NASA Handbook 4002-A (RD-05).

- Equipment External Surfaces Resistivity

Equipment external surfaces shall be sufficiently conductive, interconnected and grounded to the spacecraft structure to avoid the build-up of differential charges.

- Note: A material is considered to be conductive in an ESD sense if:

- Bulk resistivity < 109 Ωm

- Surface resistivity <109 Ω/square

Rationale:	Resistivity limits based on section 3.2 of NASA Handbook 4002-A (RD-05) definitions.

- Internal Charging

All dielectric materials and sensitive devices shall have sufficient protection such that the normally incident electron flux is less than 0.1 pA/cm2.

Rationale:	 As per section 4.1.8 NASA Handbook 4002-A guidelines (RD-05).

- Floating Conductors

Isolated conductors such as connector pins, circuit traces on printed circuit boards and radiation spot shields shall be grounded as follows:

- Isolated conductors located on or near the surface of the spacecraft structure to be grounded to the structure with less than 109 ohm so as to prevent surface charging;

Isolated conductors located inside the spacecraft structure to be grounded to the structure with less than 1012 ohm so as to prevent internal charging

except if the floating area is less than 1 cm2 and have a total capacitance less than 50 pF with respect to the structure and not be part of a multi-conductor elements structure and are more than 20 mm away from a power line.

- Notes:

Exception in accordance with ECSS-E-ST-20-06C Rev. 1 (RD-03) clause 6.3.2.a.

Multi-conductor structures exist where at least two conductive elements are separated by a dielectric surface of less than 100 cm2.

Rationale:	Based on ECSS-E-ST-20-06C Rev. 1.

## Intentional Transmitters and Receivers

- Intentional Transmitter – Transmit Power in Receive Bands

When measured at the transmit antenna port, the output power of the intentional transmitters shall not exceed -165 dBm/Hz for all intended on-board receivers’ frequency bands.

Note: Typical receiver frequency bands are:

1.560 to 1.590 GHz (Global Positioning System (GPS) L1);

1.213 to 1.243 GHz (GPS L2);

2.020 to 2.115 GHz (Telemetry, Tracking and Command (TT&C) Uplink).

Rationale:	To ensure internal electromagnetic compatibility.

- Intentional Receiver – Out of Band Rejection

All intentional receivers shall be designed to operate in the presence of out-of-band RF signals found in the surrounding environment, including emissions from the on-board transmitters.

Rationale:	To ensure electromagnetic compatibility.

- Interfering Signal Levels

All intentional receivers shall be designed to operate without degradation in the presence of ground-, sea- and space-based radars as defined in AD-16.

Note: AD-16 JSC-CR-06-070, “Space Vehicle RF Environments” provides peak on-orbit electric field strength environment for 90 ° inclination orbit.

Rationale:	To ensure electromagnetic compatibility.

- Survival

All mission critical intentional receivers shall be designed to withstand the following RF signals at the receiving antenna, without permanent degradation to performance, reliability or life:

- 126 dBµV/m from DC to 10 GHz, Cable and Wireless (CW);

- 160 dBµV/m from 1 GHz to 40 GHz, 10 microsecond pulse width at 1000 pulses per second.

- Rationale:	To avoid permanent damage in presence of intentional or non-intentional transmitters.

- Cessation of Emissions

The spacecraft shall be fitted with devices to ensure immediate cessation of its radio emissions by command whenever such a cessation is required, in accordance with the provisions of ITU RR22-1 [AD-06].

Note: In this requirement, command includes real-time, time-tagged and hardware-decoded commands.

- Rationale:	Required by international regulation.

- Cessation of Emissions Under Load Shedding

The spacecraft cessation of radio emissions shall be ensured by non-essential load shedding in the event of spacecraft attitude loss, over-current or under-voltage detection.

- Rationale:	To ensure automatic undesired transmission in case of load shedding where it could not be possible to do it by command.

## Multipaction

- Multipaction Margin

All microwave assemblies identified as being susceptible to the multipaction effect shall be designed with one of the following margins:

- Minimum 3 dB between the maximum peak RF power to which the assembly is expected to be exposed in flight and the power at which multipaction occurs, as confirmed by vacuum test over temperature;

Minimum 10 dB between the maximum peak RF power to which the assembly is expected to be exposed in flight and the power at which multipaction occurs, as determined by analysis.

Note:  RD-01contains recommendations for the design and test of RF components and equipment to achieve acceptable performance with respect to multipaction-free operations in space.

- Rationale:	To avoid any damage or performance degradation due to multipaction effect.

- Loss of Hermetic Seal by RF Units

Hermetically sealed RF units that may exhibit effects of multipaction, upon loss of the hermetic seal, shall be identified and leak rates for such RF units be demonstrated to be compatible with the design life.

- Rationale:	Loss of hermeticity will make these units susceptible to multipaction effect while the internal pressure change from 1 ATM to vacuum.

## EMC/EMI

- Spacecraft Conducted Electromagnetic Compatibility

As a minimum, the following parameters shall be assessed to ensure electromagnetic compatibility:

- Primary power and secondary power ground reference and isolation from structure;

Power line filtering (considering source impedance of the power bus);

Conducted susceptibility including common mode voltage offset tolerances.

- Rationale:	To ensure conducted electromagnetic compatibility in between the elements of the spacecraft.

- Deleted

# Acronyms, Abbreviations and Definitions

## Acronyms and Abbreviation

| Acronym | Definition |
| --- | --- |
| AD | Applicable Document |
| AIAA | American Institute of Aeronautics and Astronautics |
| AIT | Assembly, Integration and Testing |
| ANSI | American National Standards Institute |
| ATM | Atmosphere |
| C&DH | Command and Data Handling |
| CE mark | Conformity with European health, safety, and environmental standards |
| CFRP | Carbon Fibre Reinforced Polymer |
| cm | centimeter |
| COPV | Composite Overwrapped Pressure Vessels |
| CRC | Cyclic Redundancy Check |
| CSA | Canadian Space Agency |
| CW | Cable and Wireless |
| dB | Decibel |
| dBm | Decibel per milliwatt |
| DC | Direct Current |
| ECSS | European Cooperation for Space Standardization |
| EDAC | Error Detection And Correction |
| EDTS | Environmental Design and Test Specifications |
| EESS | Earth Exploration Satellite and Space Research Radio Services |
| EGSE | Electrical Ground Support Equipment |
| EMI | Electromagnetic Interference |
| EOS | Electrical Overstress |
| ESD | ElectroStatic Discharge |
| FPGA | Field Programmable Gate Array |
| FRR | Flight Readiness Review |
| GDIR | General Design and Interface Requirements |
| GHz | Giga-Hertz |
| GPS | Global Positioning System |
| Hz | Hertz |
| ICD | Interface Control Document |
| ID | Identifier |
| IR | Initial Release |
| ITU | International Telecommunication Union |
| K | Kelvin |
| LEOP | Launch and Early Orbit Phase |
| LWIR | Long Wave Infrared |
| m | metre |
| MGSE | Mechanical Ground Support Equipment |
| MLI | Multi-Layer Insulation |
| mm | millimeter |
| MMOD | Micrometeroids and Orbital Debris |
| ms | millisecond |
| MS | Margin of Safety |
| MTF | Modulation Transfer Function |
| MWIR | Mid-Wave Infrared |
| NASA | National Aeronautics and Space Administration |
| NESR | Noise Equivalent Spectral Radiance |
| pA | Pico-ampere |
| PAR | Product Assurance Requirements |
| PCU | Power Control Unit |
| PCDU | Power Control and Distribution Unit |
| pF | Pico-Farad |
| PTFE | Polytetrafluoroethylene |
| RD | Reference Document |
| RF | Radio Frequency |
| RTV | Room Temperature Vulcanizer |
| SE | System Engineering |
| SEU | Single Event Upset |
| SFd | Safety Factor – Design |
| SNR | Signal to Noise Ratio |
| SWL | Safe Working Load |
| TBD | To be determined |
| TRP | Temperature Reference Points |
| TT&C | Telemetry, Tracking and Control |
| TVAC | Thermal Vacuum |
| VNIR | Visible Near Infrared |
| WFS | WildFireSat |

## Definitions

Space Segment

The flight segment consists of everything that is on-board of the spacecraft and launcher.

Ground Segment

The ground segment consists of the ground stations and the on-ground processing necessary to provide level-1b data and ancillary data to the user segment.

Spacecraft

- An integrated assemblage of modules, subsystems, etc., designed to perform a specified mission in space.

System

- A major subdivision of the spacecraft that is viewed as a physical and functional entity for the purposes of analysis, manufacturing, testing, and recordkeeping.

Subsystem

A functional subdivision of a spacecraft consisting of two or more units.

Unit

- A functional subdivision of a subsystem and generally a self-contained combination of items performing a function necessary for the subsystem's operation.

Subassembly

- A subdivision of an assembly. Examples are wire harness and loaded printed circuit boards.

Component

- A functional subdivision of a subsystem and generally a self-contained combination of items performing a function necessary for the subsystem's operation. Examples are electronic box, transmitter, gyro package, actuator, motor, battery.

Operational Modes

- Modes in which the spacecraft can be operated. Examples are nominal, safehold, survival…

Flight Hardware

- Hardware to be used operationally in space.

Protoflight Hardware:

- Flight hardware of a new design; it is subject to a qualification test program that combines elements of prototype and flight acceptance verification; that is, the application of design qualification test levels and flight acceptance test durations.

Flight Limit Load

- Maximum load factor authorized during flight.

Qualification Load

- Load applied for qualification testing.

Design Load

- Load consisting the expected load the system is planned to support multiplied by a design safety factor.

Design Safety Factors

- Factors used to design for a greater load that was is expected in the use of the spacecraft.

Appendices

Margin Philosophy

The minimum system engineering margins for the WildFireSat flight segment development are shown in Table A-1. Additional margins are specified in the GDIR requirements. In the context of this table and as defined in the PAR (AD-01):

- “new design” refers to Category D equipment;

- “iterative design” refers to Category C equipment;

- “existing” refers to Category A, B or heritage COTS equipment.

Table A-1 – WFS Flight Segment System Engineering Margins

| Item | PDR Margin (Phase B) | CDR Margin (Phase C) | AR Margin (Phase D) |
| --- | --- | --- | --- |
| Mass | 20 % new design 10 % iterative design 5 % existing | 15 % new design 5 % iterative design 2 % existing | 5 % new design 2 % iterative design 0 % existing |
| Volume | 30 % new design 15 % iterative design 5 % existing | 10 % new design 5 % iterative design 2 % existing | 0 % new design 0 % iterative design 0 % existing |
| Propulsion (delta V) | 20 % new design 10 % iterative design 5 % existing | 15 % new design 5 % iterative design 2 % existing | 5 % new design 2 % iterative design 0 % existing |
| Margins of Safety | Refer to Section 3.3.7 |  |  |
| Mechanisms | Refer to Section 3.3.8 |  |  |
| Fatigue | Refer to AD-19 |  |  |
| Temperatures | Refer to Section 3.4.4 |  |  |
| Heaters Oversizing | Refer to Section 3.4.2 |  |  |
| Radiation | Refer to AD-19 |  |  |
| Power | 30 % new design 20 % iterative design 10 % existing | 15 % new design 10 % iterative design 5 % existing | 5 % new design 5 % iterative design 5 % existing |
| Payload performance measures (camera MTF (all bands), dynamic range (all bands), MWIR/LWIR spatial resolution, MWIR-to-LWIR co-registration accuracy, MWIR/LWIR NESR, MWIR/LWIR temperature measurement accuracy, VNIR SNR, and VNIR radiance meas. accuracy) | 30 % new design 10 % iterative design 5 % existing | 10 % new design 5 % iterative design 2 % existing | 0 % new design 0 % iterative design 0 % existing |
| Computer Resources (Software or reprogrammable Firmware based) | 50 % | 40 % | 30 % |
| Computer Resources (Non-reprogrammable Firmware based) | 30 % | 20 % | 0 % |
| Communication Bandwidth | 100 % | 50 % | 40 % |
| RF Uplink Margin* | Refer to System Requirements Document [RD-06] |  |  |
| RF Downlink Margin* | Refer to System Requirements Document [RD-06] |  |  |
| Multipaction | Refer to Section 3.12 |  |  |

* NOTE: these margins are provided for information only and are outside the scope of the GDIR.