CSA-FIN-RD-1234

Canadian Space Agency

Office of the Chief Financial Officer (OCFO)

Cost Analysis Data Requirements (CADRe) (This data is definitely 100% real) PART A # Mission : Europa Vitality Exploration (EVE) # Design Reference : # Key Decision Point : EDL-G4 # EPMO Gate : GATE-4 Draft 1 (Initial Release upon approval) September 17, 2026

This Page Intentionally Left Blank

Revision History

| Rev. | Description | Prepared by | Date |
| --- | --- | --- | --- |
| Draft 1.0 | Initial draft | SC | 17 September 2026 |

This Page Intentionally Left Blank

Table of Contents

1	Introduction	7

1.1	Purpose of this document	7

2	General Descriptive Information	7

2.1	Mission Overview	7

2.2	Context	8

2.3	Strategic Objectives	8

2.4	Selected Mission Requirements	8

2.5	High-Level Mission Architecture	9

3	Space Segment Description	11

3.1	europa oRBITER	11

3.2	Payload	18

4	Ground Segment and Mission Operations	33

5	Systems Engineering and Project Management Approach	34

5.1	TRL Levels and Heritage	34

5.2	Phasing Logic and Naming conventions	34

5.3	CSA PRoject Milestones	35

5.4	Safety and Mission Assurance	36

5.5	Qualification and Acceptance Strategy, MODEL Philosophy	36

5.6	Risk Assessment	37

5.7	Organizational Breakdown Structure	37

List of Figures

FIGURE	PAGE

Figure 1 – LiteBIRD Space Telescope	7

Figure 2 – Canadian Contribution to LiteBIRD Mission	8

Figure 3 – LiteBIRD: Exploring the very beginning of our Universe	10

List of TABLES

Table 1 – Explanation of Science Requirements	11

Table 2 -  Strategic Alignment	13

Table 3 – Select Mission Requirements	14

Table X4 – Selected Mission Requirements	Error! Bookmark not defined.

# Introduction

## Purpose of this document

CADRE Part A provides a concise, standardized synopsis of the mission or project to support costing and analysis activities. Its primary purpose is to consolidate information from multiple technical and programmatic source documents into a single, normalized reference.

Specifically, CADRE Part A:

- Summarizes key mission characteristics in a consistent and structured format

- Aggregates information drawn from multiple authoritative technical documents

- Reduces the need for analysts to repeatedly consult disparate source materials

- Ensures a common baseline description is used across analyses

- Improves efficiency, traceability, and consistency in downstream costing work

In practical terms, CADRE Part A functions as a time-saving reference document that enables analysts to quickly understand the mission context without re-interpreting or reconciling multiple source documents.

# General Descriptive Information

## Mission Overview

EUROPA VITALITY EXPLORATION (EVE) is a fictional robotic space mission designed by the Canadian Space Agency (CSA) to investigate Europa, one of Jupiter’s largest moons, and search for potential signs of extraterrestrial life. The mission will send a spacecraft to the Jovian system, where an orbiter will enter orbit around Europa and conduct detailed observations of its icy surface. Using imaging and scientific instruments, the orbiter will study Europa’s terrain and identify a suitable location for a surface landing. After completing its initial reconnaissance, the orbiter will deploy a lander toward the selected site.

Once on the surface, the lander will release a small rover to begin exploring Europa’s ice. The rover will travel across the surface, collect scientific measurements, and use a drill to penetrate the upper layers of ice. Samples obtained from below the surface will be analyzed for chemical or biological signatures that could indicate the presence of life or environments capable of supporting it. The mission’s overall goal is to improve our understanding of Europa and determine whether its hidden subsurface environment may contain evidence of biological activity.

Figure 1 – Mission Concept

Page 57

## Context

Different mission architectures have been proposed over the years. Each Design Reference Mission has its own CADRe. Make sure you are using the right file

Industry Responses

| Prime | Constellation Size | Bus | P/L |
| --- | --- | --- | --- |
| SASH TECH | 1 Sat | The Big Boy Rev3 | SashTech |

## Strategic Objectives

Consistent with the Government of Canada's Space Policy Framework, the WFS mission supports the following 2 main strategic goals:

Table 2 -  Strategic Alignment

|  | Strategic Goals | Space Strategy |
| --- | --- | --- |
| 1 | EXPLORE | Advance scientific knowledge and exploration of the Solar System. EUROPA VITALITY EXPLORATION supports the CSA’s Explore strategy by advancing Canadian scientific and technological capabilities in deep-space exploration through the investigation of Europa’s surface and subsurface environment. The mission would generate new scientific knowledge about Europa while demonstrating technologies for orbital reconnaissance, planetary landing, surface mobility, drilling, and in-situ sample analysis. |
| 2 | INSPIRE | Engage and inspire Canadians through space exploration. EUROPA VITALITY EXPLORATION supports the CSA’s Inspire strategy by using a high-profile search for potential extraterrestrial life to engage Canadians and promote interest in science, technology, engineering, and mathematics (STEM). The mission would provide opportunities for public outreach, education, and student participation, helping inspire the next generation of Canadian scientists and engineers. |

Additionally, EVE contributes to Space Diplomacy through international partnerships.

## Selected Mission Requirements

The following table presents a summary of selected mission requirements that are expected to be significant drivers of mission cost, risk, and technical complexity. For clarity and accessibility, the requirements have been consolidated, simplified, and reworded from their original form. The complete and authoritative set of requirements is contained in the Mission Requirements Document (MRD)

Table 3 – Selected Mission Requirements

| Area | Key requirement |
| --- | --- |
| Europa investigation | Characterize at least 90% of Europa's accessible surface and investigate its subsurface environment to assess the moon's geological, chemical, and biological potential. |
| Jovian transit | Transport the mission spacecraft from Earth to the Jovian system and arrive at Jupiter within 7 years of launch. |
| Europa orbit | Establish and maintain a 100–500 km circular polar orbit around Europa for a minimum of 180 days of orbital science operations. |
| Surface reconnaissance | Acquire imagery of at least 95% of Europa's accessible surface at a ground sampling distance of ≤10 m/pixel prior to landing-site selection. |
| Landing site | Identify and select a landing site with a horizontal landing uncertainty of ≤500 m, terrain slopes of ≤15°, and sufficient communication visibility to support surface operations. |
| Surface landing | Deliver the lander to the selected site and achieve a successful soft landing with a vertical velocity of ≤3 m/s and horizontal velocity of ≤1 m/s at touchdown. |
| Rover exploration | Deploy a rover capable of traversing at least 5 km across Europa's surface and conducting scientific observations at a minimum of 10 distinct investigation sites. |
| Subsurface sampling | Acquire at least 6 subsurface ice samples from depths of up to 3 m below the local surface during the primary rover mission. |
| Sample analysis | Analyze acquired samples for organic compounds, elemental composition, mineralogy, and potential biosignatures, with a minimum of 4 distinct analytical techniques. |
| Data return | Return at least 95% of mission-critical scientific and engineering data generated during the mission to Earth, with all critical data received within 30 days of acquisition. |

## High-Level Mission Architecture

Owned and operated by CSA, the mission consists of

- Government Segment: consisting of CSA resources for mission oversight, program governance, science management, and associated management costs such as travel and logistics.

- Space Segment: consisting of an internationally provided Europa orbiter and lander, and a Canadian-developed rover and scientific payload. The orbiter will perform Europa reconnaissance and support communications, while the lander will deliver the Canadian rover to the selected surface location.

- Launch Segment: consisting of an internationally provided launch vehicle and launch service provider responsible for delivering the spacecraft to the required interplanetary trajectory.

- Ground Segment: consisting of facilities, equipment, software, and personnel required to plan and command mission activities, monitor spacecraft health, receive and process telemetry, and receive, archive, and distribute scientific data.

- Operations Segment: consisting of the resources required to conduct routine mission operations, including mission planning, spacecraft and rover commanding, flight dynamics, anomaly response, scheduling, communications operations, and operational support throughout the mission lifecycle.

- Science Segment: consisting of the scientific teams, facilities, analytical resources, and data-processing capabilities required to define scientific investigations, plan observations, analyze returned data and samples, and generate scientific products and mission findings.

- Mission-Level SEITPM: consisting of the systems engineering, integration and test, and program management activities required to coordinate the mission as an integrated space system. This includes requirements management, system architecture, interface management, verification and validation, configuration management, risk management, integration and test planning, and overall program/project management.

- International Partner Segment: consisting of partner-agency resources supporting the development, integration, operation, and maintenance of the internationally provided mission elements and their interfaces with the Canadian rover and ground systems.

# Space Segment Description

The EVE space segment consists of three primary spacecraft elements: a Europa Orbiter, a Europa Lander, and a Canadian Surface Rover. The orbiter provides Europa reconnaissance, communications relay, and mission support; the lander provides controlled descent and surface deployment; and the Canadian rover performs surface mobility, subsurface sampling, and in-situ scientific investigation.

The three elements operate as an integrated system throughout the mission. During the orbital phase, the Europa Orbiter surveys the surface and identifies candidate landing sites. Following site selection, the lander separates from the orbiter and performs its descent to Europa. Once surface operations have been established, the lander deploys the Canadian rover, which conducts the primary surface and subsurface science campaign.

## europa oRBITER

The Europa Orbiter provides the primary remote-sensing and communications platform for EVE. The spacecraft is responsible for transporting the lander to the Europa vicinity, conducting orbital reconnaissance, supporting landing-site selection, and providing a communications relay between the surface assets and Earth.

Key functions of the orbiter include:

- Electrical power generation and distribution

- Three-axis attitude determination and control

- Orbital navigation and control

- Command and data handling

- High-gain communications with Earth

- Communications relay with the lander and rover

- Europa surface imaging and remote sensing

- Lander deployment

- Thermal control

- Fault management and autonomous spacecraft protection

### Structural Subsystem

The orbiter primary structure consists of a lightweight aluminum-lithium frame with carbon-fibre-reinforced polymer secondary structures. Honeycomb sandwich panels are used for equipment decks and instrument mounting surfaces.

The structure is designed to withstand launch loads, interplanetary cruise loads, Europa orbit insertion, and the mechanical loads associated with lander separation.

| Subsystem / Component | Notional Mass |
| --- | --- |
| Primary structural frame | 185 kg |
| Equipment decks and panels | 95 kg |
| High-gain antenna support structure | 32 kg |
| Solar-array deployment mechanisms | 28 kg |
| Lander separation mechanism | 46 kg |
| Instrument mounting structures | 24 kg |
| Harness and structural fittings | 38 kg |
| Structural subsystem total | 448 kg |

### Electrical power generation and distribution

The orbiter uses deployable solar arrays during the Earth-to-Jupiter cruise and Europa orbital phases. Because solar intensity at Jupiter is significantly lower than at Earth, the arrays are sized for end-of-life operation at the Jovian distance.

Two large deployable solar wings provide approximately 9.5 kW of beginning-of-life electrical power at 5.2 AU. The spacecraft power system distributes regulated power to the spacecraft bus, communications system, instruments, propulsion system, and lander interface.

Solar Array

- Two deployable solar-array wings

- Triple-junction photovoltaic cells

- Approximately 72 m² total active solar-cell area

- Beginning-of-life generation: 9.5 kW

- End-of-life generation: 7.2 kW

- Array operating voltage: approximately 120 V

- Independent array strings provide fault tolerance

Battery

A rechargeable lithium-ion battery system provides energy storage during periods of eclipse and high instantaneous power demand.

- Nominal capacity: 18 kWh

- Operating voltage: 100–120 V

- Two independent battery modules

- Battery thermal control through dedicated heaters and radiators

- Designed for approximately 2,500 equivalent deep-discharge cycles

Power Control and Distribution

The Power Control and Distribution Unit (PCDU) regulates spacecraft power and provides switched distribution to all major loads.

Functions include:

- Solar-array power regulation

- Battery charge/discharge control

- Primary and secondary power distribution

- Load switching

- Over-current protection

- Fault isolation

- Power telemetry

- Emergency load shedding

### Command an Data handling

The Command and Data Handling (CDH) subsystem provides centralized spacecraft computing, data processing, storage, command execution, and fault management.

The system consists of two redundant flight computers operating in a cold-redundant configuration.

Each computer provides:

- 64-bit radiation-tolerant processor

- 16 GB radiation-tolerant non-volatile memory

- 2 TB solid-state mass memory

- Error detection and correction

- Time-tagged command execution

- Autonomous fault detection and recovery

- Instrument command and control

- Data compression and packetization

- Software and firmware upload capability

The CDH subsystem interfaces with the spacecraft through redundant high-speed data buses. Critical spacecraft functions can continue following the loss of one flight computer.

### Thermal Subsystem

The orbiter uses a combination of passive and active thermal-control techniques to maintain spacecraft components within their allowable temperature ranges.

Thermal-control methods include:

- Multi-layer insulation

- High-emissivity radiator surfaces

- Low-emissivity thermal coatings

- Heat pipes

- Conductive thermal straps

- Localized electrical heaters

- Thermostatically controlled heater circuits

Particular thermal attention is required for the spacecraft propulsion system, batteries, instruments, and communications electronics.

The nominal spacecraft component operating range is approximately −30°C to +55°C, with survival limits extending to approximately −60°C to +70°C, depending on the individual component.

The thermal subsystem is designed to support spacecraft operations from Earth departure through Europa orbital operations.

### Attitude Determination and Control Subsystem

The orbiter uses a redundant three-axis attitude-control system to support high-resolution imaging, communications, propulsion maneuvers, and lander deployment.

Attitude determination is provided by:

- Two star trackers

- Six sun sensors

- Two inertial measurement units

- Two rate gyroscope assemblies

- One magnetometer

Attitude control is provided by:

- Four reaction wheels

- Twelve hydrazine-compatible reaction-control thrusters

- Thruster-based momentum unloading

- Autonomous safe-mode sun acquisition

The spacecraft provides a nominal pointing accuracy of ≤0.05° during science operations and ≤0.01° during high-resolution imaging.

Unlike an Earth-orbiting spacecraft, the orbiter does not use GPS for navigation. Position and velocity are determined using ground-based radiometric tracking combined with onboard optical navigation observations of Europa and Jupiter.

### Propulsion subsystem

The orbiter propulsion subsystem provides the delta-V required for interplanetary trajectory correction, Jupiter arrival, Europa orbit insertion, orbit maintenance, collision avoidance, and lander deployment operations.

The primary propulsion system uses a bipropellant chemical propulsion system consisting of monomethylhydrazine (MMH) and nitrogen tetroxide (NTO).

| Parameter | Notional Value |
| --- | --- |
| Propellant | MMH / NTO |
| Main engine thrust | 1.8 kN |
| Specific impulse | 320 s |
| Propellant load | 1,850 kg |
| Main propulsion tanks | 2 |
| Reaction-control thrusters | 12 |
| Nominal mission ΔV | 2.65 km/s |
| ΔV reserve | 15% |

The propulsion system provides the primary maneuvering capability required for Europa orbit insertion and subsequent orbital operations.

Notional ΔV Budget

| Maneuver | ΔV |
| --- | --- |
| Trajectory correction | 180 m/s |
| Jupiter orbit insertion | 1,450 m/s |
| Europa orbit insertion | 720 m/s |
| Orbit adjustment and phasing | 160 m/s |
| Lander deployment maneuver | 55 m/s |
| Collision avoidance | 40 m/s |
| Mission reserve | 395 m/s |
| Total ΔV capability | 3.00 km/s |

### TT&C

The orbiter Telemetry, Tracking and Command (TT&C) subsystem provides communications between the spacecraft and Earth and acts as the primary communications relay for the lander and rover.

The system consists of:

- One high-gain parabolic antenna

- Two medium-gain antennas

- Two low-gain antennas

- X-band command and telemetry system

- Ka-band high-rate science-data transmitter

- Redundant receivers

- Radio-frequency switching network

Earth Communications

| Parameter | Notional Value |
| --- | --- |
| Command frequency | X-band |
| Telemetry frequency | X-band |
| Science downlink | Ka-band |
| Maximum science downlink rate | 4 Mbps |
| Nominal science downlink rate | 1 Mbps |
| Command uplink rate | 128 kbps |
| High-gain antenna diameter | 3.5 m |
| Maximum transmitter RF power | 150 W |

Communications with Earth are performed through NASA/ESA/CSA-compatible deep-space ground-station infrastructure.

Surface Communications

The orbiter provides a relay link between Europa's surface assets and Earth.

- Lander ↔ Orbiter: UHF

- Rover ↔ Lander: UHF

- Rover ↔ Orbiter: UHF/S-band contingency link

- Nominal rover-to-lander data rate: 2 Mbps

- Nominal lander-to-orbiter data rate: 8 Mbps

The communications architecture allows the rover to continue collecting scientific data during periods when direct communication with Earth is unavailable.

## Payload

The Europa Orbiter payload consists of:

- One (1) high-resolution visible imaging system

- One (1) multispectral imaging system

- One (1) infrared spectrometer

- One (1) thermal infrared imager

- One (1) ice-penetrating radar

- One (1) magnetometer

- One (1) dust and particle analyzer

- One (1) payload electronics unit

The Europa Lander payload consists of:

- One (1) panoramic imaging system

- One (1) microscopic imaging system

- One (1) seismometer

- One (1) environmental sensor suite

- One (1) radiation detector

- One (1) sample-handling and analysis interface

- One (1) payload electronics unit

The Canadian Europa Rover payload consists of:

- One (1) panoramic stereo camera

- One (1) microscopic imager

- One (1) Raman spectrometer

- One (1) mass spectrometer

- One (1) X-ray spectrometer

- One (1) radiation detector

- One (1) thermal probe

- One (1) subsurface drilling and sample-acquisition system

- One (1) in-situ sample analysis chamber

- One (1) payload electronics unit

### Europa Multispectral Infrared Imagers

The proposed Europa Multispectral Infrared Imager (EMII) is a cryogenic infrared imaging instrument designed to characterize Europa's surface temperature, thermal properties, and compositional variations. The instrument combines mid-wave infrared (MWIR) and long-wave infrared (LWIR) detectors with a shared telescope assembly and selectable spectral filters.

The EMII detector assembly uses two infrared focal plane arrays (FPAs):

- Each FPA provides 640 × 512 pixels, corresponding to approximately 328,000 infrared-sensitive pixels.

- The MWIR detector operates primarily between 3.0 and 5.5 µm.

- The LWIR detector operates primarily between 7.5 and 14 µm.

- Each detector has a nominal 25 µm pixel pitch.

- The instrument provides a maximum frame rate of approximately 30 frames/s.

- A five-position filter wheel provides additional spectral discrimination within the instrument's broadband response.

- The instrument is designed for both high-resolution surface mapping and lower-resolution wide-area thermal surveys.

Notional Instrument Specifications

| Parameter | Notional Value |
| --- | --- |
| MWIR FPA resolution | 640 × 512 pixels |
| LWIR FPA resolution | 640 × 512 pixels |
| Pixel pitch | 25 µm |
| MWIR spectral range | 3.0–5.5 µm |
| LWIR spectral range | 7.5–14 µm |
| Spectral channels | 8 |
| Maximum frame rate | 30 Hz |
| Telescope aperture | 180 mm |
| Instantaneous field of view | 0.015°/pixel |
| Full detector field of view | 9.6° × 7.7° |
| Nominal orbital altitude | 250 km |
| Approximate nadir ground sampling | 65 m/pixel |
| Wide-area mapping swath | 42 km |
| Radiometric accuracy | ≤2 K |
| Operating temperature | 80–320 K scene temperature |
| Detector operating temperature | ≤70 K |
| Instrument power | 95 W nominal |
| Peak instrument power | 125 W |
| Data rate | 82 Mbps raw |
| Onboard data compression | 3:1 nominal |

The EMII is mounted on the Europa Orbiter's nadir-facing instrument deck. A two-axis pointing mechanism provides fine instrument pointing independent of the spacecraft body, allowing targeted observations of scientifically significant surface features.

The instrument's primary functions are:

- Mapping Europa's surface temperature distribution

- Identifying thermal anomalies

- Characterizing variations in surface ice properties

- Supporting geological feature identification

- Detecting potential active or recently active surface regions

- Supporting landing-site characterization

- Providing contextual data for higher-resolution visible and spectroscopic observations

At the nominal 250 km orbital altitude, the instrument provides an approximate 65 m/pixel ground sampling distance at nadir. The instrument can operate in a wide-area mapping mode or a high-resolution targeted-observation mode.

Optical Assembly

The optical system consists of an all-reflective telescope using lightweight silicon-carbide mirrors. Reflective optics are used to provide broadband infrared performance while minimizing chromatic aberration.

| Component | Notional Specification |
| --- | --- |
| Primary mirror diameter | 180 mm |
| Optical configuration | Three-mirror anastigmat |
| Effective focal length | 950 mm |
| F-number | f/5.3 |
| Optical material | Silicon carbide |
| Focal-plane assemblies | 2 |
| Filter mechanism | 5-position filter wheel |
| Calibration system | Internal blackbody + deep-space reference |

An internal calibration blackbody is used to provide periodic radiometric calibration. Deep-space observations are used as a secondary cold reference.

Mass Estimate

For the preliminary EVE spacecraft mass model, the complete EMII instrument is assigned the following notional mass:

| Item | Estimated Mass |
| --- | --- |
| MWIR detector assembly | 1.8 kg |
| LWIR detector assembly | 2.1 kg |
| Telescope and mirrors | 8.5 kg |
| Filter wheel | 1.6 kg |
| Cryogenic detector assembly | 5.2 kg |
| Instrument electronics | 3.8 kg |
| Calibration blackbody | 1.4 kg |
| Instrument structure | 4.1 kg |
| Thermal hardware | 2.7 kg |
| Harness and connectors | 1.3 kg |
| Instrument dry mass | 32.5 kg |
| Design margin | 20% |
| Allocated instrument mass | 39.0 kg |

Power Estimate

| Operating Mode | Power |
| --- | --- |
| Standby | 18 W |
| Calibration | 72 W |
| Normal imaging | 95 W |
| High-rate imaging | 110 W |
| Peak | 125 W |

The EMII is expected to operate primarily during Europa orbital passes where the spacecraft geometry provides suitable viewing conditions. Observations will be scheduled alongside the visible and spectroscopic instruments to produce co-located datasets.

Data Generation

At the maximum detector frame rate, the two FPAs can generate a substantial volume of raw data. To reduce storage and downlink requirements, the instrument electronics perform onboard:

- Bad-pixel correction

- Non-uniformity correction

- Radiometric calibration

- Image cropping

- Lossless compression

- Optional controlled-loss compression

A typical science observation is expected to generate approximately 1.5–4.0 GB of processed data, depending on observation duration and operating mode.

This makes EMII a significant contributor to the EVE orbiter's mass, power, thermal, onboard storage, and communications budgets, giving you a useful instrument to trace through the rest of your CADRe.

### VNIR Imager

The proposed Europa Visible and Near-Infrared Imager (EVNII) is a high-resolution optical imaging instrument designed to provide detailed characterization of Europa's surface. The imager will support geological mapping, landing-site characterization, identification of surface features, and contextual observations for the orbiter's other scientific instruments.

The instrument uses a radiation-tolerant CMOS focal plane coupled to a compact, wide-field telescope. A filter wheel provides selectable spectral bands across the visible and near-infrared portions of the spectrum.

| Parameter | Notional Specification |
| --- | --- |
| Instrument type | VNIR multispectral imager |
| Spectral region | 0.40–1.05 µm |
| Detector technology | Radiation-tolerant CMOS |
| Detector resolution | 4096 × 3072 pixels |
| Pixel pitch | 5.5 µm |
| Maximum frame rate | 20 Hz |
| Shutter | Global shutter |
| Spectral channels | 7 |
| Telescope aperture | 150 mm |
| Effective focal length | 1.8 m |
| Field of view | 7.2° × 5.4° |
| Nominal orbital altitude | 250 km |
| Nadir ground sampling distance | ~25 m/pixel |
| Nominal imaging swath | ~32 km |
| Radiometric accuracy | ≤5% |
| Instrument mass | 24 kg |
| Nominal power | 65 W |
| Peak power | 90 W |
| Raw data rate | ~1.5 Gbps |
| Onboard compression | 4:1 nominal |

The detector provides approximately 12.6 million pixels per image. The relatively high pixel count allows the instrument to produce detailed surface imagery while maintaining sufficient swath for regional mapping.

The VNIR instrument includes seven selectable spectral bands centered approximately at:

| Band | Central Wavelength | Primary Application |
| --- | --- | --- |
| Violet | 0.43 µm | Surface scattering |
| Blue | 0.48 µm | Ice characterization |
| Green | 0.55 µm | Surface morphology |
| Red | 0.65 µm | Geological mapping |
| Deep Red | 0.75 µm | Ice/terrain discrimination |
| Near-IR 1 | 0.90 µm | Hydrated materials |
| Near-IR 2 | 1.02 µm | Surface composition |

The seven bands are acquired using a motorized filter wheel located between the telescope and detector assembly. The system can perform sequential multispectral observations of the same surface region.

Optical Assembly

The EVNII telescope uses a compact reflective optical design optimized for high-resolution imaging at visible and near-infrared wavelengths.

| Parameter | Notional Value |
| --- | --- |
| Primary aperture | 150 mm |
| Effective focal length | 1.8 m |
| Optical configuration | Three-mirror anastigmat |
| F-number | f/12 |
| Telescope FOV | 7.2° × 5.4° |
| Optical material | Silicon carbide |
| Number of powered mirrors | 3 |
| Filter positions | 7 |
| Internal calibration target | Yes |

At the nominal 250 km Europa orbit, the instrument provides approximately 25 m/pixel resolution at nadir.

The instrument can operate in two primary modes:

Regional Mapping Mode

- Reduced spatial resolution

- Large-area surface coverage

- Multispectral imaging

- Used for geological and landing-site characterization

Targeted Imaging Mode

- Full detector resolution

- High-resolution observations of selected surface features

- Used for detailed examination of fractures, ridges, impact structures, and potential landing locations

Mass Estimate

The preliminary mass allocation for the complete VNIR instrument is:

| Item | Estimated Mass |
| --- | --- |
| CMOS detector assembly | 2.5 kg |
| Detector electronics | 2.8 kg |
| Telescope mirrors and optics | 7.0 kg |
| Filter wheel and mechanism | 1.8 kg |
| Telescope structure | 3.2 kg |
| Instrument electronics | 2.5 kg |
| Calibration assembly | 1.0 kg |
| Thermal hardware | 1.4 kg |
| Baffle and stray-light hardware | 1.2 kg |
| Harness and connectors | 0.8 kg |
| Instrument dry mass | 24.2 kg |
| Design margin | 20% |
| Allocated instrument mass | 29.0 kg |

For preliminary spacecraft budgeting, 29 kg is therefore allocated to the EVNII instrument.

Power Estimate

| Operating Mode | Power |
| --- | --- |
| Standby | 12 W |
| Calibration | 42 W |
| Normal imaging | 65 W |
| High-resolution imaging | 78 W |
| Peak | 90 W |

The instrument is expected to operate primarily during dedicated Europa observation windows. During periods of high-rate imaging, the instrument's data output is temporarily stored in the orbiter's mass-memory system before being transmitted to Earth.

Data Generation

A single full-resolution monochromatic frame contains approximately 12.6 million pixels. Assuming 14-bit detector output, one uncompressed frame produces approximately 22 MB of raw data before ancillary data and packet overhead.

At the maximum 20 Hz frame rate, the theoretical raw data generation rate is approximately 440 MB/s.

Continuous maximum-rate operation is not expected during normal mission operations. Instead, the instrument will acquire short image sequences and use onboard processing to reduce the volume of data requiring transmission.

The onboard processing chain includes:

- Dark-current correction

- Bad-pixel correction

- Flat-field correction

- Radiometric calibration

- Image co-registration

- Spectral-band selection

- Lossless compression

- Controlled-loss compression when required

A typical 30-second multispectral observation is expected to generate approximately 300–500 MB of compressed science data, depending on the selected imaging mode.

The EVNII therefore represents a significant contributor to the EVE orbiter's science data volume and onboard storage requirements, while providing the high-resolution visible and near-infrared imagery required for surface characterization and landing-site selection.

### Payload Electronics

The Payload Electronics Unit (PEU) provides the processing, data-management, instrument-control, and communications interface for the EVE scientific payload. The unit receives raw measurements from the imaging and spectroscopic instruments, performs initial processing and compression onboard, and transfers prioritized science products to the spacecraft's central data-handling system.

The PEU is designed around a radiation-tolerant heterogeneous processing architecture combining a multicore CPU with programmable logic. This allows computationally intensive operations such as image correction, spectral processing, compression, and instrument data fusion to be performed before transmission.

The preliminary EVE PEU is a notional, custom-developed flight computer rather than an existing commercial product.

| Parameter | Notional Specification |
| --- | --- |
| Product type | Radiation-tolerant payload processing computer |
| Primary processor | Quad-core 64-bit ARM processor |
| Real-time processor | Dual-core real-time processor |
| FPGA | Radiation-tolerant programmable logic device |
| CPU clock | Up to 1.5 GHz |
| Programmable logic | ~350,000 logic cells |
| RAM | 8 GB EDAC-protected memory |
| Boot memory | 2 × 256 MB radiation-tolerant NOR flash |
| Local data storage | 2 TB radiation-tolerant solid-state memory |
| Input voltage | 24–32 VDC |
| Nominal power | 55 W |
| Peak power | 80 W |
| Operating temperature | −35°C to +55°C |
| Radiation tolerance | ≥100 krad(Si) TID |
| Data interfaces | SpaceWire, Ethernet, LVDS, CAN-FD, GPIO |
| Instrument interfaces | Up to 8 high-speed instrument channels |
| Mass | 4.8 kg |
| Dimensions | 220 × 180 × 65 mm |

Onboard Processing

A major function of the PEU is to reduce the amount of raw instrument data that must be stored and transmitted by the orbiter.

The processing pipeline includes:

- Detector health monitoring

- Bad-pixel identification and correction

- Dark-current correction

- Flat-field correction

- Radiometric calibration

- Image filtering

- Image compression

- Spectral data processing

- Data prioritization

- Metadata generation

- Instrument health and performance monitoring

The PEU can operate different processing profiles depending on the instrument and mission phase. High-priority observations can be retained at full or near-full resolution, while routine survey observations can undergo more aggressive compression.

A preliminary 4:1 average reduction in science data volume is assumed for imaging instruments, with higher reductions possible for observations containing significant spatial redundancy.

Data Storage

The PEU includes approximately 2 TB of local solid-state storage. This provides temporary storage for high-rate instrument observations during periods when the spacecraft is unable to immediately transfer data to the central spacecraft computer.

The storage system uses redundant memory partitions and error-detection and correction techniques. Science data are assigned priority levels so that critical observations are retained if available storage approaches capacity.

| Data Type | Priority | Example |
| --- | --- | --- |
| Critical science | 1 | Landing-site imagery and unusual surface features |
| Primary science | 2 | Multispectral and infrared observations |
| Supporting science | 3 | Calibration and contextual imagery |
| Engineering | 4 | Instrument temperatures and health telemetry |
| Diagnostic | 5 | Debug and maintenance information |

Instrument Interfaces

The PEU provides electrical and data interfaces between the scientific instruments and the spacecraft Command and Data Handling subsystem.

The preliminary interface allocation is:

| Interface | Quantity | Nominal Data Rate |
| --- | --- | --- |
| High-speed SpaceWire | 6 | 200 Mbps/channel |
| LVDS instrument interfaces | 4 | 400 Mbps/channel |
| Gigabit Ethernet | 2 | 1 Gbps/channel |
| CAN-FD | 4 | 5 Mbps/channel |
| Discrete I/O | 32 | — |

The PEU also distributes instrument timing signals to maintain synchronization between observations from different payloads. This is particularly important when combining visible, infrared, and spectroscopic measurements of the same Europa surface feature.

Fault Management

The PEU incorporates hardware and software features intended to maintain payload operations following individual component faults.

These include:

- EDAC-protected memory

- Watchdog timers

- Current monitoring

- Automatic processor reset

- FPGA configuration scrubbing

- Redundant power inputs

- Radiation-event detection

- Instrument isolation

- Safe-mode operation

- Dual boot images

If a payload instrument experiences a fault, the PEU can electrically isolate the affected instrument while maintaining operation of the remaining payload.

Power Consumption

| Operating Mode | Power |
| --- | --- |
| Standby | 12 W |
| Instrument monitoring | 28 W |
| Normal processing | 55 W |
| High-rate processing | 68 W |
| Peak computational load | 80 W |

The PEU is expected to operate whenever one or more primary science instruments are active. During periods without science observations, the unit enters a reduced-power monitoring state.

Mass Allocation

The preliminary mass allocation for the PEU is:

| Item | Estimated Mass |
| --- | --- |
| Processor and FPGA board | 1.1 kg |
| Memory and storage modules | 0.9 kg |
| Power conditioning | 0.5 kg |
| Interface electronics | 0.6 kg |
| Radiation shielding | 0.5 kg |
| Structural enclosure | 0.7 kg |
| Thermal hardware | 0.3 kg |
| Harness and connectors | 0.2 kg |
| Estimated dry mass | 4.8 kg |
| Design margin | 20% |
| Allocated mass | 5.8 kg |

For the preliminary EVE mass budget, 5.8 kg is allocated to the Payload Electronics Unit.

The PEU therefore acts as the primary processing hub for the EVE scientific payload, reducing raw science-data volume before downlink while providing instrument control, synchronization, data storage, and fault-management functions.

### Onboard blackbody calibration

Onboard The Onboard Infrared Calibration System (OICS) provides a stable reference for maintaining the radiometric accuracy of the EVE Europa Multispectral Infrared Imager (EMII) throughout the mission.

Infrared detectors can experience changes in sensitivity due to radiation exposure, temperature variations, aging, and electronic drift. The calibration system provides the instrument with a reference source of known thermal emission, allowing the payload electronics to compare the measured detector response against the expected response and apply corrections to science observations.

The OICS consists of a small, high-emissivity calibration target positioned within the EMII optical assembly. The target is maintained at a monitored temperature using embedded heaters and temperature sensors. During calibration activities, a motorized calibration shutter moves into the optical path, allowing the detector to observe the calibration target instead of Europa.

The calibration sequence is expected to be performed:

- Before the start of major science observation campaigns

- Following significant spacecraft thermal changes

- After extended periods of instrument inactivity

- At regular intervals during orbital operations

- Following detected changes in detector performance

The calibration target is observed by both the MWIR and LWIR detector assemblies, allowing corrections to be applied independently to each spectral channel.

A secondary deep-space calibration view is also incorporated into the instrument. Observing the cold background of deep space provides an additional reference point for detector offset and low-radiance measurements.

The combination of the internal calibration target and deep-space observations allows the EMII to compensate for long-term detector drift and maintain consistent radiometric measurements over the planned mission lifetime.

Launch Segment description

The 4 spacecraft will be launched into a 550km SSO using a single dedicated Rocket Lab Electron launch.

Quoted price does not include launch insurance.

Figure 7 - Launch configuration

Alternate launch providers have been considered :

- Falcon 9 rideshare

- Starship

- Rocket Lab Neutron

- MaiaSpace

# Ground Segment and Mission Operations

The approach proposed for the baseline solution is for the Government of Canada to provide the Mission Planning System and perform operations, while EDA provided some ground segment elements.

| Mission Planning System | The Mission Planning System (MPS) will be developed and provided by the CSA. The MPS will incorporate spacecraft, lander, and rover functional requirements, operational constraints, command and telemetry databases, and science objectives. The MPS will generate mission timelines, activity schedules, and command sequences for uplink to the spacecraft. |
| --- | --- |
| Spacecraft Control System | The Spacecraft Control System (SCS) will be provided by the CSA and will provide the primary interface for spacecraft operations. The SCS will support command uplink, telemetry reception, spacecraft health monitoring, alarm management, data processing, and communications with the international orbiter and lander. |
| Receiving Stations | Government of Canada ground stations will be used in combination with commercial and international deep-space ground stations. The ground-station network will support X-band command and telemetry, Ka-band science data reception, and spacecraft tracking. The CSA will coordinate ground-station scheduling and will be the primary operational interface. |
| Spacecraft Operations | Spacecraft operations will be performed by the CSA in coordination with the international spacecraft providers. The CSA will be responsible for mission planning, spacecraft and rover commanding, health monitoring, anomaly response, communications scheduling, and coordination of orbital, landing, and surface operations. |
| Science Operations | Science operations will be performed by the CSA in coordination with Canadian and international science teams. The Science Operations Centre will support observation planning, rover investigation planning, instrument operations, scientific data processing, and selection of surface investigation sites. |
| Data Archival and Dissemination | Mission and science data will be archived and disseminated through CSA-managed data systems. Processed scientific data, including orbital imagery, rover observations, drilling results, and sample-analysis data, will be made available to the EVE science team and, following validation and applicable release requirements, to the broader scientific community. |

# Systems Engineering and Project Management Approach

## TRL Levels and Heritage

SFL bus has high heritage (TRL-9)

## Phasing Logic and Naming conventions

The following are the phases in terms of costing:

| Costing Nomenclature | Phase Description according to Costing Standards | Corresponding Name for Project team |
| --- | --- | --- |
| PREP 1 | Prototyping Activities (before Gate 1) | N/A |
| PREP 2 | Prototyping Activities (during Phase 0/A) | N/A |
| Phase 0 | Concept & Feasibility Studies | Phase 0 |
| Phase A | System Definition | Phase A |
| Phase B | Preliminary Design | Phase B |
| Phase C | Detailed Design | Phase C |
| MAIT | Manufacturing, Assembly, Integration and Test (of Canadian Systems, Subsystems and Components) | Phase D |
| ATLO | Mission Level Assembly, Test, and Launch Operations (ATLO) |  |
| Cruise | Outbound Cruise is the interplanetary journey ending with approach and orbit insertion at the destination |  |
| LEOP | Launch and Early Operations (incl. commissioning) |  |
| EDL | Entry Descent and Landing |  |
| Early Ops | For surface operations, early operations might include commissioning |  |
| Phase E | Nominal Operations (per design life) | Phase E |
| Phase X | Extended operations (mission extension) | Extended ops |
| Return and Re-Entry | Inbound cruise is the return to Earth and re-entry |  |
| Phase F | Disposal (audit) | Phase F |
| Phase G | Post-Mortem data utilization | N/A |

## CSA PRoject Milestones

The project has not created a schedule baseline yet. Assuming instrument level milestones will drive the CSA need dates.

| Instrument Phases |  | Major Milestone | Milestone Date |
| --- | --- | --- | --- |
| Phase 0 | MCR | Mission Concept Review |  |
| Phase A | SRR | System Requirements Review |  |
|  |  | Mission Rebaseline | Spring 2026 |
| Phase B | PDR | Preliminary Design Review |  |
| Phase C | CDR | Critical Design Review |  |
| MAIT | MRR | Manufacturing readiness Review |  |
|  | PSR | Pre-Ship Review of systems to Prime |  |
| SAIT | SIR | System Integration Review |  |
|  | AR | Acceptance Review |  |
|  | PSR | Pre-Ship Review of S/C to Launch site |  |
| ATLO | AR | Acceptance Review or you could use the PSR to the launch site |  |
| LEOP | L | Launch |  |
|  | CR | Commissioning Review |  |
| Cruise and EDL |  | Entry descent landing |  |
| Phase E |  | Start of Nominal Ops |  |
|  |  | End of Nominal Ops |  |
| Phase X |  | End of Life |  |
| Sample Return phase |  | When the sample arrived on Earth |  |

## Safety and Mission Assurance

Initial assessments is assumed to be Mission Class: D

## Qualification and Acceptance Strategy, MODEL Philosophy

Project Specific Quality Assurance Approach and Model Philosophy

For EVE, the qualification and acceptance approach will be tailored according to the maturity, heritage, and risk of each subsystem. Existing and previously qualified technologies will primarily follow an acceptance-testing approach, while new or mission-critical technologies will undergo qualification or protoflight testing.

Engineering models will be developed for selected high-risk elements, including the Canadian Europa Rover mobility system, drilling and sample-acquisition system, and selected payload instruments. These models will be used for functional testing, software development, operational validation, and environmental testing prior to flight hardware delivery.

The Europa Orbiter and Lander will use a combination of qualification and acceptance testing based on the international partners’ existing hardware heritage. Previously qualified units will generally undergo acceptance testing at the unit level, while new or significantly modified units will follow a qualification approach.

The Canadian rover will use a protoflight approach for the flight unit, supported by engineering models for high-risk mechanical and scientific functions. The rover drill and sample-handling system will undergo additional qualification testing using representative Europa ice simulants.

At the spacecraft level, the baseline approach will be protoflight testing for the orbiter, lander, and rover. Environmental testing will include vibration, shock, thermal-vacuum, electromagnetic compatibility, and functional testing. Where practical, testing will be performed at the integrated spacecraft level to verify interfaces between the mission elements.

The qualification and acceptance program will focus on identifying design and interface issues as early as possible while avoiding unnecessary duplication of testing on flight hardware. Particular emphasis will be placed on the rover drilling system, communications interfaces, thermal performance, planetary protection requirements, and long-duration autonomous operations.

The project will prioritize early development and testing of high-risk systems so that sufficient time remains for corrective actions before system-level integration. All qualification and acceptance activities will be documented through configuration-controlled test plans, procedures, results, and verification reports.

The baseline model philosophy is therefore:

| Hardware | Model Philosophy |
| --- | --- |
| Europa Orbiter | Protoflight |
| Europa Lander | Qualification / Protoflight |
| Canadian Europa Rover | Protoflight with Engineering Models |
| Rover Drill | Qualification |
| Scientific Payloads | Qualification / Acceptance |
| Payload Electronics | Acceptance with qualification testing of new designs |
| Ground Systems | Engineering Models and Acceptance Testing |
| Mission Software | Engineering/Operational Validation Models |

## Risk Assessment

The project team shall produce a risk register (ref CADRe part C)

## Organizational Breakdown Structure

Earth Daily Analytics’ supply chain

Science Instruments:
Northern Optics Research (Ottawa, Canada) – VNIR and infrared imaging instruments

Payload:
Laurentian Space Systems (Montreal, Canada) – integrated scientific payload and sample-analysis equipment

Payload Electronics:
Orbital Electronics Canada (Toronto, Canada) – payload electronics unit and instrument interfaces

Rover:
Canadian Planetary Robotics (Calgary, Canada) – Europa surface rover, mobility system, drilling and sample acquisition

Lander:
European Planetary Systems Agency (Darmstadt, Germany) – Europa lander and descent system

Orbiter:
Nordic Deep Space Systems (Kiruna, Sweden) – Europa orbiter and communications relay

Launch:
International Launch Services (Florida, USA) – heavy-lift launch vehicle and launch services

Ground Segment:
Canadian Space Operations Centre (Longueuil, Canada) – mission planning, spacecraft operations and data processing

Prime:
Canadian Space Agency (Longueuil, Canada)